## sensor_three_phase_BLDC.c File Reference

### Detailed Description

Atmel Corporation

- File : sensor_three_phase_BLDC.c

- Compiler : IAR EWAAVR 2.28a/3.10c

- Support mail : avr@atmel.com

- Supported devices : All devices with 3 PWM channels and Pin Change Interrupts can be used. The example is written for ATmega48.

- AppNote : AVR443 - Sensor based control of three phase brushless DC motor

- Description : Example of how to use an ATmega48 to control a brushless DC motor using Hall sensor for rotor position feedback.

Revision

1.1

Date

Wednesday, June 15, 2005 07:38:42 UTC

#include <ioavr.h>

#include <inavr.h>

#include "sensor_three_phase_BLDC.h"

### Functions

- void Init_ADC (void)

Initialize ADC module.

- void Init_MC_pin_change_interrupt (void)

Initialize pin change interrupts for PORTB pin 1, 2 and 3.

- void Init_MC_timers (void)

Initialize motor control timers.

- void main (void)

Application starts here.

- __interrupt void PCINT0_ISR (void)

Pin Change Interrupt Service Routine.

- void Set_Direction (unsigned char direction)

Set motor direction, CW og CCW.

- void Set_Speed (unsigned char speed)

Set motor speed, in PWM duty cycle.

### Variables

- __regvar __no_init unsigned char count

Optimized variable decremented every pin change int.

- unsigned char drvPatternsCCW []

CCW rotation patterns.

- unsigned char drvPatternsCW []

CW rotation patterns.

- __regvar __no_init union _fastTemp fastTemp

Used for optimized temporary varables.

- __regvar __no_init unsigned char hallMask

Workaround for internal compiler error.

- __regvar __no_init unsigned char * pDrvPattern

Stores the current motor driver pattern.

### Variable Documentation

#### unsigned char drvPatternsCCW[]

Initial value: {

0,

PDP2_CCW,

PDP4_CCW,

PDP3_CCW,

PDP6_CCW,

PDP1_CCW,

PDP5_CCW,

COM0P2_CCW,

COM0P4_CCW,

COM0P3_CCW,

COM0P6_CCW,

COM0P1_CCW,

COM0P5_CCW,

COM2P2_CCW,

COM2P4_CCW,

COM2P3_CCW,

COM2P6_CCW,

COM2P1_CCW,

COM2P5_CCW

}

Configuration of pin drive levels and timer COM bits in different phases for CounterClockWise rotation.

#### unsigned char drvPatternsCW[]

Initial value: {

0,

PDP2_CW,

PDP6_CW,

PDP1_CW,

PDP4_CW,

PDP3_CW,

PDP5_CW,

COM0P2_CW,

COM0P6_CW,

COM0P1_CW,

COM0P4_CW,

COM0P3_CW,

COM0P5_CW,

COM2P2_CW,

COM2P6_CW,

COM2P1_CW,

COM2P4_CW,

COM2P3_CW,

COM2P5_CW

}

Configuration of pin drive levels and timer COM bits in different phases for ClockWise rotation.

## sensor_three_phase_BLDC.h File Reference

### Detailed Description

Atmel Corporation

- File : sensor_three_phase_BLDC.h

- Compiler : IAR EWAAVR 2.28a/3.10c

- Support mail : avr@atmel.com

- Supported devices : All devices with 3 PWM channels and Pin Change Interrupts can be used. The example is written for ATmega48.

- AppNote : AVR443 - Sensor based control of three phase brushless DC motor

- Description : Example of how to use an ATmega48 to control a brushless DC motor using Hall sensor for rotor position feedback.

Revision

1.1

Date

Wednesday, June 15, 2005 07:38:42 UTC

### ADC multiplexing

- #define ADC_MUX_SHUNT_H  ((0 << REFS1) | (0 << REFS0) | (1 << ADLAR) | (1 << MUX2))

- #define ADC_MUX_SHUNT_L  ((0 << REFS1) | (0 << REFS0) | (1 << ADLAR) | (1 << MUX2) | (1 << MUX0))

- #define ADC_MUX_SPEED_REF  ((0 << REFS1) | (0 << REFS0) | (1 << ADLAR))

### PWM output configuration (for both TC0 and TC2)

- #define COM0P1_CCW  ((UL_ON)|(VL_OFF)|(1<<WGM01)|(1<<WGM00))

- #define COM0P1_CW  ((UL_OFF)|(VL_OFF)|(1<<WGM01)|(1<<WGM00))

- #define COM0P2_CCW  ((UL_ON)|(VL_OFF)|(1<<WGM01)|(1<<WGM00))

- #define COM0P2_CW  ((UL_OFF)|(VL_OFF)|(1<<WGM01)|(1<<WGM00))

- #define COM0P3_CCW  ((UL_OFF)|(VL_ON)|(1<<WGM01)|(1<<WGM00))

- #define COM0P3_CW  ((UL_OFF)|(VL_ON)|(1<<WGM01)|(1<<WGM00))

- #define COM0P4_CCW  ((UL_OFF)|(VL_ON)|(1<<WGM01)|(1<<WGM00))

- #define COM0P4_CW  ((UL_OFF)|(VL_ON)|(1<<WGM01)|(1<<WGM00))

- #define COM0P5_CCW  ((UL_OFF)|(VL_OFF)|(1<<WGM01)|(1<<WGM00))

- #define COM0P5_CW  ((UL_ON)|(VL_OFF)|(1<<WGM01)|(1<<WGM00))

- #define COM0P6_CCW  ((UL_OFF)|(VL_OFF)|(1<<WGM01)|(1<<WGM00))

- #define COM0P6_CW  ((UL_ON)|(VL_OFF)|(1<<WGM01)|(1<<WGM00))

- #define COM2P1_CCW  ((WL_OFF)|(1<<WGM21)|(1<<WGM20))

- #define COM2P1_CW  ((WL_ON)|(1<<WGM21)|(1<<WGM20))

- #define COM2P2_CCW  ((WL_OFF)|(1<<WGM21)|(1<<WGM20))

- #define COM2P2_CW  ((WL_ON)|(1<<WGM21)|(1<<WGM20))

- #define COM2P3_CCW  ((WL_OFF)|(1<<WGM21)|(1<<WGM20))

- #define COM2P3_CW  ((WL_OFF)|(1<<WGM21)|(1<<WGM20))

- #define COM2P4_CCW  ((WL_OFF)|(1<<WGM21)|(1<<WGM20))

- #define COM2P4_CW  ((WL_OFF)|(1<<WGM21)|(1<<WGM20))

- #define COM2P5_CCW  ((WL_ON)|(1<<WGM21)|(1<<WGM20))

- #define COM2P5_CW  ((WL_OFF)|(1<<WGM21)|(1<<WGM20))

- #define COM2P6_CCW  ((WL_ON)|(1<<WGM21)|(1<<WGM20))

- #define COM2P6_CW  ((WL_OFF)|(1<<WGM21)|(1<<WGM20))

- #define HP1  ((1<<HALL1)|(0<<HALL2)|(1<<HALL3))

- #define HP2  ((1<<HALL1)|(0<<HALL2)|(0<<HALL3))

- #define HP3  ((1<<HALL1)|(1<<HALL2)|(0<<HALL3))

- #define HP4  ((0<<HALL1)|(1<<HALL2)|(0<<HALL3))

- #define HP5  ((0<<HALL1)|(1<<HALL2)|(1<<HALL3))

- #define HP6  ((0<<HALL1)|(0<<HALL2)|(1<<HALL3))

- #define UL_OFF  ((0<<COM0A1)|(0<<COM0A0))

- #define UL_ON  ((1<<COM0A1)|(0<<COM0A0))

- #define VL_OFF  ((0<<COM0B1)|(0<<COM0B0))

- #define VL_ON  ((1<<COM0B1)|(0<<COM0B0))

- #define WL_OFF  ((0<<COM2B1)|(0<<COM2B0))

- #define WL_ON  ((1<<COM2B1)|(0<<COM2B0))

### Port control registers for Hall sensor inputs.

- #define DDR_HALL  DDRB

- #define PIN_HALL  PINB

- #define PORT_HALL  PORTB

### Port control registers for drive stages.

- #define DDR_MC  DDRD

- #define PIN_MC  PIND

- #define PORT_MC  PORTD

### Hall sensor input pins.

- #define HALL1  PB1

- #define HALL2  PB2

- #define HALL3  PB3

- #define HALL_MASK  ((1<<HALL1)|(1<<HALL2)|(1<<HALL3))

### Output pins to drive stages.

- #define MC_MASK  ((1<<UL)|(1<<VL)|(1<<WL)|(1<<UH)|(1<<VH)|(1<<WH))

- #define UH  PD7

- #define UL  PD6

- #define VH  PD4

- #define VL  PD5

- #define WH  PD2

- #define WL  PD3

### CCW driving configuration

IO driving configuration (driving enabled by setting IO to output)for counter clockwise rotation.

- #define PDP1_CCW  ((1<<UL)|(0<<VL)|(0<<WL)|(0<<UH)|(1<<VH)|(0<<WH))

- #define PDP2_CCW  ((1<<UL)|(0<<VL)|(0<<WL)|(0<<UH)|(0<<VH)|(1<<WH))

- #define PDP3_CCW  ((0<<UL)|(1<<VL)|(0<<WL)|(0<<UH)|(0<<VH)|(1<<WH))

- #define PDP4_CCW  ((0<<UL)|(1<<VL)|(0<<WL)|(1<<UH)|(0<<VH)|(0<<WH))

- #define PDP5_CCW  ((0<<UL)|(0<<VL)|(1<<WL)|(1<<UH)|(0<<VH)|(0<<WH))

- #define PDP6_CCW  ((1<<UL)|(0<<VL)|(1<<WL)|(0<<UH)|(1<<VH)|(0<<WH))

### CW driving configuration

IO driving configuration (driving enabled by setting IO to output)for clockwise rotation.

- #define PDP1_CW  ((0<<UL)|(0<<VL)|(1<<WL)|(0<<UH)|(1<<VH)|(0<<WH))

- #define PDP2_CW  ((0<<UL)|(0<<VL)|(1<<WL)|(1<<UH)|(0<<VH)|(0<<WH))

- #define PDP3_CW  ((0<<UL)|(1<<VL)|(0<<WL)|(1<<UH)|(0<<VH)|(0<<WH))

- #define PDP4_CW  ((0<<UL)|(1<<VL)|(0<<WL)|(0<<UH)|(0<<VH)|(1<<WH))

- #define PDP5_CW  ((1<<UL)|(0<<VL)|(0<<WL)|(0<<UH)|(0<<VH)|(1<<WH))

- #define PDP6_CW  ((1<<UL)|(0<<VL)|(0<<WL)|(0<<UH)|(1<<VH)|(0<<WH))

### Defines

- #define ADC_REF_VOLTAGE  5.0f

- #define CLOCKWISE  1

Used by Set_Direction() for CW rotation. Used by Set_Direction() for CCW rotation.

- #define COUNTERCLOCKWISE  0

- #define CURRENT_SHUNT_RESISTANCE  0.22f

- #define MAX_CURRENT  1.0f

Max motor current in Amperes. Current shunt resistanse in Ohms. ADC reference voltage in Volts.

- #define MAX_CURRENT_ADC  (signed int)(MAX_SHUNT_VOLTAGE / VOLTS_PER_ADC_STEP)

- #define MAX_SHUNT_VOLTAGE  (MAX_CURRENT * CURRENT_SHUNT_RESISTANCE)

- #define PATTERN_COM0_OFFSET  6

- #define PATTERN_COM2_OFFSET  12

- #define PATTERN_DRV_OFFSET  0

Offset for drive pattern in table. Offset for Timer 0 Output Compare config pattern in table. Offset for Timer 2 Output Compare config pattern in table.

- #define VOLTS_PER_ADC_STEP  (ADC_REF_VOLTAGE / 256)

Volts per LSB with 8-bit ADC. Max current shunt voltage. Calc max value for raw ADC reading.