***************************************************************

*	      How to load PCL-711 Device Driver into	      *

*	      BASICA and QuickBASIC 4.0, 4.5		      *

***************************************************************

To provide additional programming language support, the PCL-711

offers a software driver that is for QuickBASIC 4.0 and 4.5

programming.  The file name of this driver is "711QB.QLB" and it

is available in your PCL-711 software diskette.

The programming syntax of the QuickBASIC driver is exactly the

same as the BASICA driver described in the manual.  However, there

are minor changes regarding the loading and initialization of the

QuickBASIC driver.

Loading the Driver

In BASICA, the routine to load PCL-711 driver is

such as:

110 'LOAD 711BAS.BIN DRIVER TO BASIC AREA

120 CLEAR ,57344!

130 DEF SEG=0

140 SG=256*PEEK(&H511)+PEEK(&H510)

150 SG=SG+57344!/16

160 DEF SEG=SG

170 BLOAD "711BAS.BIN",0

180 'END OF DRIVER LOADING

In QuickBASIC, since the PCL-711 driver is treated as a library,

it is not necessary to load PCL-711 driver to the work area as it

is in BASICA.  Please eliminate the above coding from program

originally written for BASICA before using it for QuickBASIC. To

make the PCL-711 driver ready for use, users must key in the

following command when entering QuickBASIC.

QB filename /L 711QB.QLB

Initializing the Driver

In BASICA, the routine to initialize PCL-711 driver is such as:

100 PORT% = &H300

110 PARAM%(0) = 0	   'Board number

120 PARAM%(1) = PORT%

**   140 PCL711 = 0	 'DRIVER OFFSET

160 FUNC% = 3

170 CALL PCL711(FUNC%, PARAM%(0))

180 IF ERR% <>0 THEN PRINT "INSTALLATION FAILED !":STOP

In QuickBASIC, the PCL-711 driver is treated as a library, users

do not have to specify the PCL-711 driver offset. Please delete

this following line from the initialization routine.

140 PCL711 = 0

If the user wants to run the program examples offered with PCL-711

in QuickBASIC, the routine section to load "711BAS.BIN" and

the statement "PCL711=0" must be deleted. Besides, a statement

such as:

DIM DTA%(N), CH%(N)

must be changed to a statement as

DIM DTA%(500), CH%(500)

The size of these arrays can be smaller or larger and it must be

a constant otherwise QuickBASIC will indicate "Overflow" error.