************************************************************************

*             How to compile and link C or C++ application             *

*             program with the PCL-711 Device Driver                   *

************************************************************************

1. How to compile and link with the driver for Microsoft C  ?

Example for SMALL mode :

cl  /AS  /c  711demo.c

link 711demo + 711CS.LIB;

Note :    /AS for SMALL Mode

/AM for MEDIUM Mode

/AC for COMPACT Mode

/AL for LARGE Mode

2. How to compile and link with the driver for Turbo C  ?

Example for SMALL mode :

1. In Integrated Development Environment

. Set project name (*.prj) in project options

Example : 711.prj

. 711.prj file contents :

711demo.c

711cs.lib

. Select Compile \ Build all

2. In DOS Command Use TCC Program

. TCC   -ms  711demo.c  711cs.lib

Note :    -ms for SMALL Mode

-mm for MEDIUM Mode

-mc for COMPACT Mode

-ml for LARGE Mode

3. How to compile and link with the driver for Borland C++  ?

Example for SMALL mode :

BCC   -ms 711demo.c 711cs.lib

Note :    -ms for SMALL Mode

-mm for MEDIUM Mode

-mc for COMPACT Mode

-ml for LARGE Mode