- CHAPTER 2

- INSTALLATION AND COMMISSIONING

- CONTENTS

PART 1

INSTALLATION	3

General	3

General Installation Notes	3

Transceiver Unit Installation	4

Cable Details	5

AC Supply Cables	5

Power Consumption	5

Switch Fuses	5

Control Cables	5

Co-Axial Cables	6

I-Band Waveguide	7

Siting	7

Earthing and Bonding	7

Cabling Information	8

Cable Schedules:	9

Transceiver Control Cable	9

Radar Silence Control	10

Turning Unit Enable	10

Bearing Input Signals (4096 ppr)	11

Buffered Bearing Outputs	11

Remote Indication Outputs	11

AC Mains Input	11

Motor Supply	12

Transceiver Configuration	13

PART 2	17

COMMISSIONING	17

Introduction	17

Pre Switch-on Checks	17

Live Testing	17

Commissioning Checklist	20

- Illustrations

- Figure 2.1 Transceiver Mounting Alternatives	4

- Figure 2.2 Connector Identification	8

- Figure 2.3 Input Board Configuration	13

- Figure 2.4 Trigger Board Configuration	15

This page intentionally blank

- PART 1

# INSTALLATION

## General

The Installation Engineer is responsible for the provision of the work required for the installation, and making sure that the work is done by the Contractor.

- Preparation

- Preparation for installation is as follows:

(1)	If necessary, the structure of the ship should be strengthened to support the components of the system.

(2)	The provision and fitting of mounting facilities for the components of the system.

(3)	The provision of power supplies, terminated by a switch-fuse box. The Installation Engineer will specify the voltage level and current rating of the supplies.

(4)	The provision and installation of cable trays, battens and/or conduits for the interconnecting cables of the system.

(5)	The provision of all earthing and bonding leads or strips.

## General Installation Notes

- The Installation comprises the following operations:

(1)	If required the Contractor will install the mast and any necessary stays.

(2)	The unpacking and external inspection of the components of the system.

(3)	The installation of the Transceiver Unit (and where applicable the Scanner Unit).

(4)	The installation of waveguide brackets and/or deck glands.

(5)	The installation of the other components of the system.

(6)	Planning and laying of cables.

## Transceiver Unit Installation

The Bulkhead Transceiver 65825C is installed below decks in a suitable location, such as the ship’s equipment room. The installation should pay due regard to accessibility for maintenance and servicing, and the distance between the Transceiver and the Scanner Unit. For maximum performance, this should be kept as short as practicable, as the loss per metre of the I-Band waveguide is typically 10dB per 100 metres, i.e. 2dB for 20 metres. This loss applies to both the transmitted RF pulse and the received signals.

When using the cable types specified in this manual, the maximum permissible separation between the Turning Unit and the Transceiver Unit is 30 metres, and that between the Transceiver and the Display is 67 metres.  Special “low-loss” cables will be required to guarantee reliable operation over greater distances, and it is recommended that the customer checks with the Systems and Products Department at Litton Marine Systems when contemplating such installations.

When choosing the installation location for the Transceiver, due regard must be made for the routing of the I-Band waveguide.

The Transceiver may be attached to the bulkhead by several methods. The actual method chosen will depend on individual circumstances, but due regard must be given to the likely vibration and shock loading which may be experienced. The available methods include through bolting to the bulkhead, or mounting on studs provided by the shipyard.

The fixing holes provide for M8 bolts or studs at 240mm X 765 mm centres.

Figure 2.1  Bulkhead Transceiver - Mounting Alternatives

## Cable Details

Refer to the Cabling Schedules section below for details of cables and related terminations. The Inter-Unit Cabling diagrams show the interconnections to be made for the different systems available. For details of the I-Band RF Feeder see Page 2-7.

## AC Supply Cables

The physical size of the cable used to connect the AC supply of the Ship to the radar should consider the following points:

(1)	The voltage drop from the supply to the radar must not exceed 2% of the supply voltage.

(2)	Make sure that the current required by the radar does not exceed the current rating of the cable.

If the radar is to be supplied from a three phase supply, the radar must be connected between two of the three phase lines and have an isolating transformer to provide a voltage at the display input of 220/240 V.

## Power Consumption

The power consumption of the radar is given in the Technical Specification of the radar.

## Switch Fuses

The radar is to be connected to the power supply of the Ship via a Switch Fuse. The Switch Fuse is not supplied with the radar, and is therefore of local manufacture. The fuse rating of the Switch Fuse is given in the System Fuses section of this Chapter.

## Control Cables

The control cables detailed on the Inter-Unit Wiring diagrams may be used up to a maximum length of 67m, unless otherwise stated.

All multicore cables except the DATA cable, conform to IEC 92/3 and DEF STAN 61-12 Part 5.  The cables all have multiple cores of tinned copper stranded wire conductors. Each conductor is insulated with PVC to form cores. The cores are screened with braided tinned copper wire, and the complete cable is sheathed with PVC. The temperature range of the cables is -25C to +85C.

- The DATA cable conforms to UL Type CL2, AWM 2919 and CSA PCC FT4.

Control Cables (continued)

| Service Code | Cable Diam. mm | No. of Cores | Core - Strands /Diam. mm | Core CSA mm2 | Ohms /Km (at 20C) | Current Rating A | Rated Volts (RMS) V | Decca Marine Code Number |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 16-2-2C | 6.9 | 2 | 16/0.2 | 0.5 | 40.1 | 2.5 | 440 | 3218376 |
| 16-2-4C | 7.7 | 4 | 16/0.2 | 0.5 | 40.1 | 2.5 | 440 | 3209342 |
| 16-2-6C | 8.7 | 6 | 16/0.2 | 0.5 | 40.1 | 2.5 | 440 | 3211274 |
| 16-2-12C | 11.0 | 12 | 16/0.2 | 0.5 | 40.1 | 2.5 | 440 | 3211266 |
| 37-3-2R | 10.3 | 2 | 37/0.32 | 2.88 | 6.79 | 13 | 440 | 3228207 |
| 37-3-4R | 11.8 | 4 | 37/0.32 | 2.88 | 6.79 | 13 | 440 | 3214044 |
| DATA * | 8.4 | 4 pairs | 7/0.2 | 0.23 | ---- | ---- | 30 | MA00007419 |

- CABLE CORE COLOUR ABBREVIATIONS

- Multicore Cables

| R | Red | W | White | V | Violet |
| --- | --- | --- | --- | --- | --- |
| B | Blue | BK | Black | P | Pink |
| G | Green | BN | Brown | L/G | Light Green |
| Y | Yellow | O | Orange | GY | Grey |

- DATA cable (4 Twisted Pairs)

| B/W | Blue/White | Twisted with | W/B | White/Blue |
| --- | --- | --- | --- | --- |
| O/W | Orange/White | Twisted with | W/O | White/Orange |
| G/W | Green/White | Twisted with | W/G | White/Green |
| BN/W | Brown/White | Twisted with | W/BN | White/Brown |

Note:	Blue/White is predominately Blue with a narrow white trace, and White/Blue is predominately White with a narrow Blue trace.

Similarly with the other pairs.

## Co-Axial Cables

The signal cables (co-axial) detailed on the Inter-Unit Cabling diagrams may be used up to a maximum length of 67m. These cables are double screened, and have a nominal impedance of 75.

| Service Code | Cable Diameter mm | Nominal Impedance  | Attenuation at 10 MHZ dB/100 m | Comments |
| --- | --- | --- | --- | --- |
| PT1YM | 6.0 | 75 | 3.9 | Double screened, with Mylar insulation between screens |

## I-Band Waveguide

The recommended waveguide is an elliptical R.F. Feeder capable of operating up to 9.8GHz. This has a minimum bend radius of 200mm (8") in the E Plane and 480mm (19") in the H Plane. It may be used up to a maximum length of 30m.

| Service Code Number | Size (over jacket) mm | Maximum Twist degrees/m | Attenuation at 9.4GHz dB/100 m | Comments |
| --- | --- | --- | --- | --- |
| 6803652 | 33.5 x 23 | 3 | 10 | Andrews Type EW85 |

## Siting

The site and method of mounting the below-decks equipment should be chosen to comply, as far as possible, with the wishes of the owner or master of the vessel. The following points should also be considered:

(1)	Compass safe distance.

(2)	Ease of access for servicing.

(3)	Positioning to avoid the ingress of moisture.

(4)	Positioning away from doors or ports that are frequently used.

(5)	Positioning to minimise accidental damage.

(6)	Environmental considerations such as; adequate ventilation, dust, dirt, clean air.

(7)	Local vibration environment.

## Earthing and Bonding

The common earth point of the power supply of the Ship must be connected to the chassis (case) of the radar unit receiving the supply. Use the crimp tags at the cable entry points to make this earth connection.

The object of a bonded system is to ensure that the exposed metal case or chassis of each piece of equipment is brought to a common earth potential. Bonding is needed both to achieve EMC (electromagnetic compatibility) and in some instances to prevent stray currents causing electrolytic action.

The effectiveness of bonding is dependent on ensuring a very low resistance and inductance of the bonding cables or straps to ground. Bonding connections must also be protected from corrosion. The most suitable materials for bonding connections between the stainless steel earthing bolt and steel, are aluminium or tinned copper straps. For the aluminium strap it is essential to ensure that the contact areas are oxide-free, and that they are coated with a thin layer of conducting paste before assembly to seal the joint and provide a large contact area. A zinc-plated washer should be used at the stainless steel end of the connection. If a tinned copper strap is connected directly to aluminium, an aluminium washer should be used between the strap and the aluminium.

The method of connecting the bonding cables/straps together is very important. The connections should be made from each piece of equipment to a common connection point (ship’s earth) - not looped from unit to unit.

A 6mm stainless steel earth bolt is provided on each unit of the system for the bonding connection. No other connection must be made to the earth bolt as the bonding earth is a non current carrying connection.

## Cabling Information

Fig 2.2 – Connector Identification

The layout of the Input Board 65825820 is shown above with the connector areas highlighted. Refer to the following cabling schedules for details of the inter-connections,

Access to the Input Board is made by removal of the Transceiver lid, and then the lower cover.

Note:  In the following Cable schedules, reference is made to the cable screens being terminated at the earth tags.  The transceiver is supplied with DIN cable glands with integral earth cones, and where the prescribed cables are used this will be a sufficient earth connection.  Otherwise, the earth tags MUST be used.

## Cable Schedules:

| ### Transceiver Control Cable |  |  |  |
| --- | --- | --- | --- |
| The control signals to and from the transceiver are all in RS422 format, and are inter-connected using screened twisted pair data cables. | TxRx | NAME | FUNCTION |
|  | TSD1 | DU DATA+ | RS422 twisted pair carrying Display Data to Transceiver |
|  | TSD2 | DU DATA- |  |
|  | TSD3 | TX DATA+ | RS422 twisted pair carrying Transceiver Data to Display |
|  | TSD4 | TX DATA- |  |
|  | TSD5 | TX TRIG+ | RS422 twisted pair carrying Display Triggers to Display |
|  | TSD6 | TX TRIG- |  |
|  | E/TAG | SCREEN |  |

|  | 267 | DATA CABLE (4 T/PAIRS) |  |  |
| --- | --- | --- | --- | --- |
| When the display used in this installation is a BridgeMaster E, then the complete wiring schedule becomes: | TxRx | FUNCTION | CLR | DU |
|  | TSD1 | DU DATA+ | B/W | TSA1 |
|  | TSD2 | DU DATA- | W/B | TSA2 |
|  | TSD3 | TX DATA+ | O/W | TSA3 |
|  | TSD4 | TX DATA- | W/O | TSA4 |
|  | TSD5 | TX TRIG+ | G/W | TSA5 |
|  | TSD6 | TX TRIG- | W/G | TSA6 |
|  | E/TAG | EARTH (0V) | SCR | E/TAG |

|  | 268 | DATA CABLE (4 T/PAIRS) |  |  |
| --- | --- | --- | --- | --- |
| For installations using a BridgeMaster E Interswitch Unit, then the wiring schedule becomes as opposite, (where ‘x’ is either ‘A’ or ‘B’ for a 65842A Interswitch, or from ‘A’ to ‘F’ in the case of the 65846A Interswitch | TxRx | FUNCTION | CLR | ISW |
|  | TSD1 | DU DATA+ | B/W | TSTx1 |
|  | TSD2 | DU DATA- | W/B | TSTx2 |
|  | TSD3 | TX DATA+ | O/W | TSTx3 |
|  | TSD4 | TX DATA- | W/O | TSTx4 |
|  | TSD5 | TX TRIG+ | G/W | TSTx5 |
|  | TSD6 | TX TRIG- | W/G | TSTx6 |
|  | E/TAG | EARTH (0V) | SCR | E/TAG |

|  | 269 | DATA CABLE (4 T/PAIRS) |  |  |
| --- | --- | --- | --- | --- |
| For installations using a Display Compatibility Unit to couple the transceiver to another display type then the Cable Schedule becomes: | TxRx | FUNCTION | CLR | DCU |
|  | TSD1 | DU DATA+ | B/W | TSB1 |
|  | TSD2 | DU DATA- | W/B | TSB2 |
|  | TSD3 | TX DATA+ | O/W | TSB3 |
|  | TSD4 | TX DATA- | W/O | TSB4 |
|  | TSD5 | TX TRIG+ | G/W | TSB5 |
|  | TSD6 | TX TRIG- | W/G | TSB6 |
|  | E/TAG | EARTH (0V) | SCR | E/TAG |

| ### Radar Silence Control |  |  |  |
| --- | --- | --- | --- |
| When the Silence Controlling input is an RS422 signal, this should be wired between TSD7 & TSD8. For RS423 signals, use TSD7 (signal) & TSD10 (return). For pulse or contact inputs, then use TSD9 (signal) & TSD10 (return) | TxRx | NAME | FUNCTION |
|  | TSD7 | SILENCE+ | RS422/3 |
|  | TSD8 | SILENCE- | RS422- |
|  | TSD9 | SILENCE B | Pulse/contact |
|  | TSD10 | 0V | Return |
|  | E/TAG | EARTH (0V) |  |

| ### Turning Unit Enable |  |  |  |
| --- | --- | --- | --- |
| The Turning Unit Enable signal is used to control the rotation of the antenna during radar transmission. It can be over-ridden by linkage. | TxRx | NAME | FUNCTION |
|  | TSD11 | 0V | 0V |
|  | TSD12 | CONTACTOR SUPPLY | +12V |
|  | TSD13 | TU ENABLE. | TU EN |
|  | E/TAG | EARTH (0V) |  |

|  | 270 | Turning Unit Enable 16-2-2C – 2core cable |  |  |
| --- | --- | --- | --- | --- |
| When the Turning Unit is a Bulkhead BridgeMaster E unit, 65801B series, the wiring schedule becomes: | TxRx | FUNCTION | CLR | TU |
|  |  | Not used | R |  |
|  | TSD13 | TU ENABLE | B | TSB10 |
|  | E/TAG | EARTH (0V) | SCR | E/TAG |

|  | 271 | #### Performance Monitor 16-2-12C - 12 core cable |  |  |
| --- | --- | --- | --- | --- |
| When the BridgeMaster E Bulkhead Turning Unit 65801B series has an integral Performance Monitor Unit, then a composite 12-core cable may be used to connect both the Performance Monitor signals and the azimuth and heading marker signals as per this schedule: | TxRx | FUNCTION | CLR | TU |
|  | TSH1 | PM ON/OFF | R | TSC1 |
|  | TSH2 | PM TX/RX | B | TSC2 |
|  | TSH3 | PM TUNE | G | TSC3 |
|  | TSH4 | XR ADJUST | Y | TSC4 |
|  | TSH5 | XT ADJUST | W | TSC5 |
|  | TSH6 | +12V PM | BK | TSC6 |
|  | TSH7 | +12V PM | BN | TSC7 |
|  | TSH8 | 0V | V | TSC8 |
|  | TSH9 | -12V PM | O | TSC9 |
|  | TSH10 | -12V PM | P | TSC10 |
|  | TSG1 | AZI PULSES | L/G | TSC11 |
|  | TSG3 | HEADING MKR | GY | TSC12 |
|  | E/TAG | EARTH (0V) | SCR | E/TAG |

| ### Bearing Input Signals (4096 ppr) |  |  |  |
| --- | --- | --- | --- |
| ##### Facilities for connecting the transceiver to turning units with a variety of signal formats is provided on TSG. Configuration to accept the signal type is contained in the configuration data in this chapter. | TxRx | NAME | FUNCTION |
|  | TSG1 | AZI PULSES+ | RS422, RS423 or Logic input |
|  | TSG2 | AZI PULSES- |  |
|  | TSG3 | HMKR I/P+ | RS422, RS423 or Logic input |
|  | TSG4 | HMKR I/P- |  |
|  | TSG5 | +5.1V (DIG) | +5V SUPPLY |
|  | TSG6 | 0V | +5V RETURN |
|  | E/TAG | EARTH (0V) | SCREEN |

| ### Buffered Bearing Outputs |  |  |  |
| --- | --- | --- | --- |
| ##### Two identical sets of buffered bearing and heading marker output signals are generated, both in RS422 format, at connectors TSC and TSE. (TSE Pinout same as TSA) | TxRx | FUNCTION |  |
|  | TSC1 | AZI PULSES OUT+ | RS422 o/p signals, Use twisted pair |
|  | TSC2 | AZI PULSES OUT- |  |
|  | TSC3 | HMKR OUT+ | RS422 o/p signals, Use twisted pair |
|  | TSC4 | HMKR OUT- |  |
|  | E/TAG | EARTH (0V) | Screen |

| ### Remote Indication Outputs |  |  |  |
| --- | --- | --- | --- |
| For customer applications, six separate volt-free outputs are generated using solid state relays. Open circuit voltage must not exceed 40volts, and closed circuit current not to exceed 0.5amp. | TxRx | FUNCTION |  |
|  | TSF1 | TxRx ON |  |
|  | TSF2 | TxRx ON RETURN |  |
|  | TSF3 | READY TO TX |  |
|  | TSF4 | READY TO TX RETURN |  |
|  | TSF5 | TRANSMITTING |  |
|  | TSF6 | TRANSMITTING RETURN |  |
|  | TSF7 | SP IN USE |  |
|  | TSF8 | SP IN USE RETURN |  |
|  | TSF9 | MP IN USE |  |
|  | TSF10 | MP IN USE RETURN |  |
|  | TSF11 | LP IN USE |  |
|  | TSF12 | LP IN USE RETURN |  |
|  | S/TAG | EARTH (0V) |  |

| ### AC Mains Input |  |  |  |
| --- | --- | --- | --- |
|  | 272 | ##### AC Mains Supply |  |
| ##### The A.C. Mains Supply is connected via the two part connector PLA, mounted on the lower face of the transceiver. | PLA | FUNCTION |  |
|  | A | AC LINE |  |
|  | B | AC NEUTRAL |  |
|  | C | EARTH |  |

Customer Variant Options

| ### Motor Supply |  |  |  |
| --- | --- | --- | --- |
| ##### For some versions of the transceiver a Motor supply PSU is included, generating an adjustable output at TSA. | TxRx | NAME | FUNCTION |
|  | TSA1 | MOTOR SUPPLY+ | +25 to +60V |
|  | TSA2 | MOTOR SUPPLY- | Supply return |
|  | E/TAG | EARTH (0V) | SCREEN |

|  | 208 | #### Motor Supply 37-3-2R - 2 Core cable |  |  |
| --- | --- | --- | --- | --- |
| When the Turning Unit is a Bulkhead BridgeMaster E unit, 65801B series, the wiring schedule becomes: | TxRx | FUNCTION | CLR | TU |
|  | TSA1 | MOTOR SUPPLY+ | R | TSA1 |
|  | TSA2 | MOTOR SUPPLY- | B | TSA2 |
|  | E/TAG | EARTH (0V) | SCR | E/TAG |

## Transceiver Configuration

Configuration for required mode of operation is achieved by setting switches and links as follows:

Input Board 65825820

Figure 2.3 - Input Board Configuration

The component layout of the Input Board 65825820 is shown above, with the configuration areas highlighted

Links 1 & 2		Not Fitted (for service use only)

Link 5			Link Pins 1 - 2 for normal operation, link 2 – 3 to override

Switch Settings:

S1:

1 to 4 OFF, 5 to 8 ON for normal operation (I/O connections via TSD)

1 to 4 ON, 5 to 8 OFF for test mode (I/O connections via TSB)

S2:	[This is used for selection of azimuth & heading marker input signal options. Sections1 to 3 are for the azimuth input, 4 to 6 are for heading marker]

| Input type | 1 | 2 | 3 | Notes Connect to TSG1 & 2 ##### Connect to TSG1 only ##### Connect to TSG1 only |
| --- | --- | --- | --- | --- |
| RS422 | OFF | OFF | ON |  |
| RS423 | OFF | ON | OFF |  |
| Logic | ON | OFF | OFF |  |
| No input | OFF | ON | OFF |  |

| Input type | 4 | 5 | 6 | Notes Connect to TSG3 & 4 ##### Connect to TSG3 only ##### Connect to TSG3 only |
| --- | --- | --- | --- | --- |
| RS422 | OFF | OFF | ON |  |
| RS423 | OFF | ON | OFF |  |
| Logic | ON | OFF | OFF |  |
| No input | OFF | ON | OFF |  |

S3:	[This is used to configure the Silence Input Interface]

| Input type | 1 | 2 | 3 | 4 | Notes Connect to TSD7 & 8 ##### Connect to TSD7 only ##### Connect to TSD9 only ##### Connect to TSD9 only |
| --- | --- | --- | --- | --- | --- |
| RS422 | ON | OFF | OFF | OFF |  |
| RS423 | OFF | OFF | OFF | ON |  |
| Pulse | OFF | ON | ON | OFF |  |
| Contact | OFF | ON | ON | OFF |  |
| No input | OFF | ON | OFF | ON |  |

Trigger Board 65825801

The component layout of the Trigger Board 65825801 is shown in Figure 2.4, with the configuration areas highlighted

The Link settings are as follows:

Links 1 and 7 are for test use only, LK1 always fitted, LK7 never fitted.

LK2	Set to position 2-3 for Pre-trigger 1 always present,

or in position 1-2 for operation while in transmit only

LK3	Set to position 2-3 for Pre-trigger 2 always present,

or in position 1-2 for operation while in transmit only

LK4	Set to position 2-3 for Pre-trigger 3 always present,

or in position 1-2 for operation while in transmit only

LK5	Heading marker selection. Position 2-3 selects external, position 1-2 selects an internally generated (synthetic) one.

LK6	Azimuth pulse selection. Position 2-3 selects external, position 1-2 selects an internally generated (synthetic) one.

LK8	Wobbulation enable.  Position 1-2 is disable, 2-3 (or missing) for enable

LK11	Tune select. Position 1-2 is normal, 2-3 selects the local tune control RV5.

LK12	AFC selector. Position 1-2 is normal, 2-3 selects Manual, 3-4 selects AFC only.

LK14	EPROM selector.  The link assembly is not normally fitted

Figure 2.4 Trigger Board Configuration

Switch S1 settings

| Pos. | ON | OFF |
| --- | --- | --- |
| 1 | Timer Bypassed | Timer in use |
| 2 | Heading Marker Over-ride | Heading Marker In Use |
| 3 | Radar Silence Normal | Radar Silence Inverted |
| 4 | Heater Turndown Disabled | Heater Turndown Active |
| 5 | Timer Short (90secs) | Timer Long (180secs) |
| 6 | Not used at present |  |
| 7 | Baud rate X4 | Baud rate X1 |
| 8 | Baud rate X2 | Baud rate X1 |

This page intentionally blank

# PART 2

# COMMISSIONING

## Introduction

Prior to switch on, Initialisation and Commissioning involves ensuring that the complete installation has been carried out in accordance with the information contained in this chapter, together with that contained in the Installation Diagrams provided with the individual units. If a discrepancy exists between this Manual and the Installation Diagrams, the latter should take precedence.

Initialisation and Commissioning also includes ensuring that all of the connections and links have been correctly set prior to switch on.

## Pre Switch-on Checks

1.	Are all the Units securely attached to their mountings, properly earthed and bonded, and sited with regard to the recommendations given at the beginning of this chapter? In particular, is the turning circle for the antenna free of obstructions?

2.	Are all cables correctly installed with due regard to strain relief and protection against chafe?

3.	Are all cables correctly terminated according to the cabling schedules, and are their braids clamped where applicable?

4.	Have the correct fuses been installed in the Switch/Fuse units and the Transceiver?

5.	Have the links and switches in the Transceiver been set to the required operational configurations? The Commissioning Checklist should be used to confirm this.

- If a Display Compatibility Unit is installed within the system, have the links been correctly set as shown in the relevant manual?

## Live Testing

- Apply power to the System.  Check that the “Mains on” indicator only is illuminated (steady or flashing – see Chapter 8) on the transceiver, but the ‘Transceiver on’ indicator does not light up.  Remove Transceiver lid (if still fitted).

WARNING:- TAKE EXTREME CARE WHEN THE EQUIPMENT IS OPENED FOR ACCESS, LETHAL VOLTAGES ARE EXPOSED.

- Switch on the Display (or control station).  Check that the “Transceiver on” indicator on the transceiver is now illuminated, and that the Display does not indicate a “comms” error.  (In a system using a DCU, check that LEDs 3, 4 & 5 on the DCU are extinguished).

- Check that after 90 seconds (or 180 for a BridgeMaster II) the Display warming up message is removed and the system states that it is in standby.

- Check that the antenna is clear to rotate, the isolating switches for the motor volts are all closed, and that the radar site is cleared of personnel.

- Select transmit, and confirm that the antenna commences to rotate, and after two revolutions the Transmitting indicator comes on.

- If it is required to operate the transceiver in a “stand alone” mode, then ensure that links LK11 & LK12 are fitted in position 2-3.  With the transceiver operating in long pulse, monitor the voltage on R4. Starting from fully anti-clockwise, gradually increase RV5.  The value on R4 should increase towards 2V and then dip sharply to a null before rising again.  Adjust RV5 to retrieve the null setting.  Remove link LK12, and check that the null is retained.  Park LK12 in position 3-4.

- If the system is using a BridgeMaster II Display and a DCU, then proceed from paragraph 14.  Otherwise continue from para 16.

- DCU setting up information.  Apart from switch configurations, the DCU requires the tuning potentiometer(s) to be adjusted for manual tuning.

15.	BridgeMaster II Display

WARNING:- TAKE EXTREME CARE WHEN THE EQUIPMENT IS OPENED FOR ACCESS, LETHAL VOLTAGES ARE EXPOSED.

Remove the Display Unit top cover, and the Display Compatibility Unit cover.  Refer to Figure 3.36 in 65600009, for location of RV3, display coarse tune control.

- Set the radar to 24 nautical.mile range, using the RANGE keys.

- Set the radar to transmit mode Long Pulse using the PULSE key.

- Set the TUNE control on the display control panel to the central position.

- Select AFC off.

- Inside the BridgeMaster Display, set RV3 (coarse TUNE) control fully counter clockwise (zero) position.  If no Performance Monitor setting is required, replace the Display Unit top cover.

- On the Display Compatibility Unit adjust RV1 coarse tune until the TUNE INDICATOR shows the maximum number of diamond symbols. (In an inter-switched system where the DCU can be connected to more than one transceiver, adjust RV1 to tune transceiver A, RV2 to tune transceiver B etc..)

- Replace Display Compatibility Unit cover.

- If an I-Band Performance Monitor is included within the Scanner Unit, then refer to the BridgeMaster Ancillaries Manual for setting up procedure.

- Replace the BridgeMaster Display cover.

16.	Working down the Trigger Board from the top, first set the local oscillator to the required operating frequency (RV1).

- Then set the prf limiter potentiometers on each pulse length by turning anticlockwise until the division drops, then turn back two turns clockwise. RV2 adjusts short pulse, RV3 medium and RV4 long.

- Now adjust the pre-trigger pulses for delay and width to suit requirements.  The potentiometer references are:

Pre-Trigger 1:	Delay RV7	Width RV8

Pre-Trigger 2:	Delay RV9	Width RV10

Pre-Trigger 3:	Delay RV11	Width RV12

(Note: If insufficient lead time can be obtained with a delay potentiometer set at minimum (fully anti-clockwise), then adjust RV6 clockwise to obtain the required timing and repeat setting up of all other pre-triggers if previously set)

19.	Finally confirm the selection of the link options on each pre-trigger pulse, either for operation whilst transmitting only (position 1-2) or in standby as well (2-3).

20.	Record all actions and settings on the Installation and Commissioning Record Sheet, which appears at the end of this Chapter.

# Commissioning Checklist

- Input Board Settings

| FUNCTION | Switch Element | SETTING |
| --- | --- | --- |
| Azimuth Type | S2 Slider 1 |  |
|  | S2 Slider 2 |  |
|  | S2 Slider 3 |  |
| HMKR Type | S2 Slider 4 |  |
|  | S2 Slider 5 |  |
|  | S2 Slider 6 |  |
| Silence Input | S3 Slider 1 |  |
|  | S3 Slider 2 |  |
|  | S3 Slider 3 |  |
|  | S3 Slider 4 |  |

| FUNCTION | FUNCTION | SETTING | CCT REF |
| --- | --- | --- | --- |
| Rotation | Controlled |  | LK5 1-2 |
|  | Continuous |  | LK5 2-3 |

- Trigger Board Settings

| FUNCTION | FUNCTION | SETTING | CCT REF | Monitor Point |
| --- | --- | --- | --- | --- |
| Pre Trigger 1 | Lead | s | RV7 | SKYL |
|  | Width | s | RV8 | SKYL |
|  | #### Pre Trigger in Stby | Yes | LK2 2-3 |  |
|  | #### In transmit only | No | LK2 1-2 |  |
| Pre Trigger 2 | Lead | s | RV9 | SKYM |
|  | Width | s | RV10 | SKYM |
|  | Pre Trigger Stby | Yes | LK3 2-3 |  |
|  | In transmit only | No | LK3 1-2 |  |
| Pre Trigger 3 | Lead | s | RV11 | SKYN |
|  | Width | s | RV12 | SKYN |
|  | Pre Trigger Stby | Yes | LK4 2-3 |  |
|  | In transmit only | No | LK4 1-2 |  |
| Internal PRF |  | Hz | RV1 | TP4 |
| Wobbulation | On |  | LK8 2-3 |  |
|  | Off |  | LK8 1-2 |  |
|  | Wobbulation adjust | s | RV13 | (NNF) |
| PRf Limiters | Long Pulse | s | RV4 | TP6 |
|  | Medium Pulse | s | RV3 | TP6 |
|  | Short Pulse | s | RV2 | TP6 |
| Tuning | External |  | LK11 1-2 |  |
|  | Internal (RV5) |  | LK11 2-3 | LK11 pin 2 |
| AFC/Manual | #### External |  | LK12 1-2 |  |
|  | #### Permanent Manual |  | LK12 2-3 |  |
|  | ##### Permanent AFC |  | LK12 3-4 |  |

3.	Trigger Board Configuration Switch S1

| Element | Function | Select | Disable | Setting |
| --- | --- | --- | --- | --- |
| 1 | Bypass Timer | ON | OFF |  |
| 2 | Heading Marker Over-ride | ON | OFF |  |
| 3 | Radar Silence Polarity | TRUE | INV |  |
| 4 | Heater Turndown Disable | ON | OFF |  |
| 5 | Short Timer (90 sec) | ON | OFF |  |
| 6 | not used at present | x | x |  |
| 7 | Baud rate | see | below |  |
| 8 | Baud rate | see | below |  |

| Baud Rate | S1/7 | S1/8 | Setting |
| --- | --- | --- | --- |
| 9,600 baud | OFF | OFF |  |
| 19K2 baud | OFF | ON |  |
| 38K4 baud | ON | OFF |  |
| 76K8 baud | ON | ON |  |

The commissioning checklist indicates the Test point (TP) numbers or output socket to be used for monitoring the various settings.

4. 	Trigger Board Potentiometer settings

These are not possible to set to specific values until the transceiver is operational.  Likewise it is not possible to define the setting by visual inspection. [However when a trigger board is delivered from the factory as a spare it should be already set to operate at a 3.3KHz clock frequency and short pulse interval, and medium and long pulse arranged to divide down by two and four respectively.  It is therefore quite safe to run in a transceiver prior to the customer specific settings being made.]

This page intentionally blank