- CHAPTER 6

- CONTENTS

PART 1 ~ SERVICING

Servicing Policy	6.3

Contact	6.3

Replacing Component Parts

Access to Replaceable Parts	6.5

Replaceable Parts List	6.6	Replaceable Part Locations	6.7	Magnetron Replacement and Setting	6.8	Microwave Assembly Removal	6.10	Circulator Replacement	6.11	Limiter Replacement	6.11	Receiver Assembly Replacement	6.11	Receiver Filter Pcb Replacement	6.11	Trigger Pcb Replacement and Setting	6.13	Transceiver Sub Assembly Removal	6.17	Power Supply Pcb Replacement and Setting	6.18	Modulator Pcb Replacement and Setting	6.20	Video Buffer Pcb Replacement and Setting	6.22	Input Pcb Replacement and Setting	6.22	Mains Filter Assembly Replacement	6.25	Motor Supply Pcb Replacement and Setting.	6.26

Illustrations

Figure 6.1 – Service Access to Transceiver.	6.5	Figure 6.2 – Location of Major Components.	6.7	Figure 6.3 – Magnetron Replacement.	6.9	Figure 6.4 – Removal of Microwave Assembly	6.10	Figure 6.5 – Removal of Microwave Sub-Assemblies	6.11	Figure 6.6 – Removing Trigger PCB	6.13	Figure 6.7 – Trigger Board Configuration Details	6.14	Figure 6.8 – Trigger Board Test Point Locations	6.15	Figure 6.9 – Removal of Transceiver Sub-assembly.	6.17	Figure 6.10 – Removal of Power Supply PCB	6.18	Figure 6.11 – Power Supply Component Layout.	6.19	Figure 6.12 – Removal of Modulator PCB	6.20	Figure 6.13 – Modulator Component Layout.	6.21	Figure 6.14 – Video Buffer and Input Board Removal	6.22	Figure 6.15 – Video Buffer Component Layout.	6.23	Figure 6.16 – Input Board Component Layout.	6.23	Figure 6.17 – Removal of Mains Input Filter Assembly	6.25	Figure 6.18 – Removal of Motor Supply PCB	6.26	Figure 6.19 – Motor Supply Board Component Layout	6.27

- CHAPTER 6

- CONTENTS (Continued)

PART 2 ~ FAULT FINDING and DIAGNOSTICS

FAULT FINDING

General	6.29

Error Messages	6.30

Tellbacks	6.32

Use of Fault Finding Flow Charts	6.34

Diagnostic Sheets

Sheet 1 – Power Supply Faults.	6.35

Sheet 1a - Power Supply Faults.	6.37

Sheet 2 – Rotation Faults.	6.39

Sheet 3 – Azimuth Data Faults for RS422 Input.	6.41

Sheet 4 – Missing Trigger, No Transmission, Spark-Gap Errors.	6.43

Sheet 5 – Video, Transmitter Failure to Transmit.	6.45

Sheet 5a – Receiver Faults.	6.47

Sheet 5b – Inhibit Tellback Error.	6.49

Sheet 6 – Serial Communication Faults.	6.51

Sheet 7 – External and Pre-Trigger Faults.	6.53

Sheet 8 – Mod H.T. Incorrect.	6.55

Sheet 8a – Magnetron Current Incorrect.	6.57

Sheet 9 – Tellback Errors.	6.59

- CHAPTER 6 ~ PART 1

SERVICING

# SERVICING POLICY

The transceiver is a modular design, and defective or suspect modules can be simply isolated and replaced using the following illustrations and notes.  The fault diagnosis procedures are contained in Part 2 of this Chapter

Fault diagnosis to component level is not usually possible without specialised test equipment.

Some of the PCBs are assembled using surface mount techniques.

Service repair is therefore by module replacement only

The module may be covered by the Service Exchange scheme

# CONTACT

If a unit exhibits a fault, please contact your supplier or local dealer, or if on international trade, contact:

Litton Marine Systems B.V.	Telephone:	+(44)(0) 208 329 2400

SERVICE CONTROL	   Or	+(44)(0) 208 329 2000

Burlington House

118 Burlington Road	Fax:	+(44)(0) 208 329 2458

New Malden

Surrey  KT3  4NR	Telex:	261545 RACDECCA G

England

For general non-urgent enquiries, the following e-mail addresses are also available:

gmk01@nml.litton-marine.com	rjp01@ nml.litton-marine.com

It is recommended that any e-mails are sent to both of the above addresses.

Information required for Service.

- Name of Vessel ( and Satcom or Fax Number if fitted)

- Equipment Type, including prefix and suffix letters

- Software version number (where applicable)

- Next port of call, ETA and Ship’s agents

- Fault description (as detailed as possible)

- Contact name.

### This page intentionally blank

CAUTION.

Once the Transceiver Lid is removed, hazardous voltages are exposed

Service Access to Transceiver

Figure 6.1

List of Replaceable Parts

Item	Part Number	Illustration

Magnetron	#See Items List	6.3

Receiver Assembly 	#See Items List	6.4

Receiver Filter PCB	65801818	6.5

Circulator 	#See Items List	6.5

Limiter 	#See Items List	6.5

Trigger Board	*65825801/811	6.6

Transceiver Sub-Assembly	65825635	6.9

Power Supply Board	65825807	6.10

Modulator Board	65825802	6.12

Input Board	65825820	6.14

Video Buffer PCB	65825805	6.14

Mains Input Filter PCB	65825822	6.17

Motor Supply Board	65825806	6.18

Notes:

# These items are frequency conscious and vary between the transceiver types.

* This item varies in Part Number according to firmware fitted

Location of Major Components

Figure 6.2

Magnetron Replacement Notes:

When running up a replacement magnetron the following procedure is recommended:

- Set RV1 on the power supply to fully anticlockwise to minimise current.

- Allow transceiver to warm up for 15 to 20 seconds in standby before selecting transmit.

- Select transmit initially in Long Pulse.  Then over a period of 3 to 5 minutes gradually increase the setting of RV1 until the voltage at TP100 agrees with the value recorded on the label on the transformer.

- Now select medium pulse and leave to stabilise for a further minute before finally checking performance in short pulse.

Adopting the above procedure should maximise the life expectancy of the new magnetron.

Magnetron Replacement.

Figure 6.3

Removing Microwave Assembly

Figure 6.4

Removal of Microwave Sub Assemblies

Figure 6.5

This page intentionally blank

Replacement of Trigger PCB

Figure 6.6

Trigger Board Configuration Details

Figure 6.7

Trigger Board Test Point Locations

Figure 6.8

Additional notes for removal of Transceiver Sub-Assembly

Prior to removing the sub-assembly, ensure that the transceiver is isolated from the mains supply.

Starting from the top, disconnect the leads to the microwave assembly.

They are the magnetron leads, the earth strap, the receiver lead (PLYA) and the limiter drive lead (PLYD).

Next disconnect the two ribbon cables to PLYB and PLYK, and unthread them from the sub-assembly so that they hang free.

Now remove all eight coaxial leads from the trigger board and allow to drop down clear of the sub-assembly.

Unplug the mains lead from PLTA on the power supply

In the lower left hand corner, up to three of the ribbon leads must be removed from the power supply.

They are PLTC (6-way), PLTF (2-way), and for a 65825CA only, PLTG (3-way)

Note:  There are two 2-way leads coming from the Power Supply Unit, PLTE and PLTF (both feeding cooling fans) and if in doubt as to which to remove, disconnect both.  One lead will come out attached to the sub-assembly and can be re-inserted into the power supply after removal of the sub-assembly.

Finally slacken the four fixing screws and slide the sub-assembly upwards before lifting clear of the base fixings.

When replacing the sub-assembly, check that all leads are clear and cannot be trapped, re-insert ALL leads in the reverse order to the removal, and finally SCREW DOWN the 4 fixing screws firmly.

Double check the system before restoring power.

Removing Transceiver Sub–Assembly.

Figure 6.9

Removing Power Supply PCB

Figure 6.10

Power Supply Configuration and Test Points

Figure 6.11

Removing Modulator PCB

Figure 6.12

Modulator Component Layout

Figure 6.13

Video Buffer and Input Board Removal

Figure 6.14

Video Buffer Component Layout

Figure 6.15

Input Board Component Layout

Figure 6.16

This page intentionally blank

Removal of Mains Input Filter Assembly

Figure 6.17

Removal of Motor Supply PCB

Figure 6.18

Motor Supply PCB Component Layout

Figure 6.19

This page intentionally blank

- CHAPTER 6  ~ PART 2

FAULT FINDING.

# WARNING

Lethal voltages are exposed when the Transceiver covers are removed. Only qualified persons should work on the equipment when power is applied.

ALWAYS isolate the unit from the mains supply when removing or replacing the cover.  To make the Transceiver safe it is necessary to isolate it from the mains supply, it is not adequate to turn the unit off, as the PFC section of the Power Supply which is not mains isolated is active whenever the mains is connected to the unit.

General.

Before using the following fault finding flow chart visually check the unit to ensure that there are no loose plugs or connections.

All major power supply lines can be monitored at the Input Pcb, see Table 1 below.

| # TABLE 1 |  |
| --- | --- |
| # Location – Input Pcb |  |
| # Test Point | # Valid Readings |
| +29V | +27V - +31V |
| +18V | +17V - +19V |
| +12V | +11.3V - +12.6V |
| +5V | +4.6V - +5.25V |
| -12V | -11.3V - -11.6V |
| Mod HT | +0.9V - +1.5V |

For the Power Supply to operate, the DU Data+ and DU Data- signal lines must be active. It is not necessary for serial data to be present, the presence of a DC voltage >2.5V between the lines will start the Power supply.

(A test link LKA on Power Supply PCB when set to position 1 – 2 will force the Power Supply to operate independently of the DU Data input.  This link should only be used in this position as a service aid, and must be reset afterwards to position 2 – 3 for normal operation.)

Provision is made on the Input Pcb for local control of the transceiver using a PC loaded with Test Software and having an RS422 adapter (Transceiver Test Set 65825EA).

For Testing 65825CA and 65825CD, this will run at 38.4Kbaud, using “NIS02”.

For Testing 65825CB or CC, this will run at 9.6Kbaud, using “NIS01”.

- For local control, set the dual-in-line switch S1 as follows:

| Operating Mode | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Normal | OFF | OFF | OFF | OFF | ON | ON | ON | ON |
| Local Control | ON | ON | ON | ON | OFF | OFF | OFF | OFF |

Connect the PC to TSB as follows

Data Out+		TSB1

Data Out-		TSB2

Data In+		TSB3

Data In-		TSB4

[Note:  remember to reset S1 after operation under local control]

ERROR MESSAGES.

The following errors are reported by the transceiver:

MESSAGE FAILURE. (Diagnostic Sheet 6 Entry Point “L”.)

The transceiver expects to receive a message from the controlling device at least once per second, if the message rate drops below this the transmitter is switched to Standby and a Message Fail error is signalled. The Transmitter will remain in Standby until the next message is received which contains a ‘transmit request bit’ set to transmit.

Possible causes:

Controlling device failure.

Inter unit cabling.

Inter Pcb cabling in transceiver.

Defective Input Pcb. (signal is only looped through)

Defective Trigger Pcb.

CORRUPT DATA. (Diagnostic Sheet 6 Entry Point “L”.)

The transceiver checks the parity of each character in an incoming message, and verifies the checksum for each complete message. If sufficient corrupted messages are received a Corrupt Data error is returned, and the transceiver remains in the state it was in when it received the last valid message. Normal operation will be resumed, and the Corrupt Data error will be cleared when the next valid message is received.

Possible causes:

Controlling device failure.

Bad connections or earth bonding on inter unit cabling between Transceiver and controlling device.

Inter Pcb cabling in transceiver .

Defective Input Pcb. (signal looped through only.

Defective Trigger Pcb.

SPARK DETECT. (Diagnostic Sheet 4 Entry point “G”.)

The Modulator incorporates a spark gap to protect the pulse transformer from the excessive voltage that is generated if the magnetron mis-fires.

If the spark gap operates for more than two seconds this is detected and Spark Detect error is signalled, and the Transmitter is switched to Standby for 1 minute.

After one minute the Transmitter will revert to transmit if the transmit request bit is set for transmit.

Possible causes;

Magnetron instability at end of life.

Missing magnetron heaters, due to Modulator Pcb failure,  failure of

Magnetron Heater Supply from the Power Supply Pcb.

Inter Pcb cable from Power Supply to Modulator Pcb.

Defective Modulator Pcb.

Defective trigger Pcb.

HMKR FAILURE.  (Diagnostic Sheet 3 Entry Point “C”)

When operating normally the transmitter will not transmit unless the antenna is rotating.

Antenna rotation  is confirmed by checking for the presence of the Heading Marker.

If the Heading Marker is detected as missing but azimuth pulses still present the Transceiver will be switched to Standby and the HMKR Failure error is returned.

Transmission can only be restored after a Heading marker failure by switching the Transceiver off and back on.

The above function will be disabled if the HMKR Override switch (S1-2 ) on the Trigger Pcb is set to ON. If the switch is set to ON the Heading Marker error signal is suppressed.

Possible causes:

Inter unit cabling Transceiver to Scanner unit.

Defective Digitiser or Pulse Bearing Board in Turning Unit.

Incorrectly configured Input Pcb.

Defective Input Pcb.

MOD TRIG FAIL, CHARGE TRIG FAIL . (Diagnostic Sheet 4 Entry Point “E”.)

The Trigger Pcb generates the Modulator, and Charge Triggers. If either pulse is detected as missing a Mod Trig Fail, or Charge Trig Fail error is returned as applicable.

Possible causes;

Defective Trigger Pcb

BIST DATA

The following information is transmitted from the transceiver to the controlling device.

| Parameter | Normal Range | Diagnostic Sheet - /- Entry Point |
| --- | --- | --- |
| Magnetron Current | 5.5A – 9.5A | Sheet 8 / Entry Point Q. |
| +12V | +11V - +13.4V | Sheet 1 / Entry Point “A” |
| +30V | +26V - +36V | Sheet 1 / Entry Point “A” |
| MOD HT | -500V - -680V | Sheet 8 / Entry Point “R |

BIST data is only approximate and must not be used for making adjustments.

A deviation from the above values indicate a fault condition.

TELLBACKS.

Within the serial data transmitted by the transceiver are a number of status tellbacks to indicate the Transceiver status. If this data is monitored and displayed by the controlling device it is possible to check that the transceiver status is as requested.

Transmit.

This bit is set if the controlling device requests transmit and:

The transceiver has completed its two minutes warm up

The Heading Marker has not failed.

There is not a Message Fail error.

There is not a Spark Gap error.

Inhibit

This bit is set whenever ‘Transmit’ is requested by the controlling device and the transceiver is inhibited because the Radar Silence input is active. The sense of the Radar silence input can be selected by DIL switch S1 part 3 on the Trigger Pcb.

Medium Pulse and Short Pulse.

These bits reflect the status of the pulse length control signals within the transceiver.

They indicate the status as follows;

| Selection | SP Bit | MP Bit | Receiver Bandwidth |
| --- | --- | --- | --- |
| Short Pulse | 1 | 0 | Wide |
| Medium Pulse | 0 | 1 | Wide |
| Long Pulse | 0 | 0 | Narrow |
| (Illegal Code) | 1 | 1 | Undefined |

Wide-band.

This bit indicates the bandwidth selected for the receiver, when set to “1” “wideband” is selected.

The selected bandwidth is linked to the pulse-width as shown in the table above.

AFC On.

This bit indicates the tuning mode selected for the receiver. When set to “1” AFC mode is selected. (For some transceivers, this bit may be permanently set to “1” as a result of internal link settings.)

Timer

This bit indicates the status of the magnetron “warm up” timer within the transceiver. It is set to “0” for a preset period after the transceiver is turned on.

The timer allows the magnetron cathode to reach its operating temperature before the HT is applied.

Warm up time delay can be selected to be either 90 seconds, or 180 seconds by DIL switch S1 part 5 on the Trigger Pcb.

After the warm up delay, if the transceiver is turned off then provided it is turned back on within 10 seconds the timer will be overridden, allowing immediate return to the transmit condition.

The timer can be inhibited by DIL switch S1 part 1 on the trigger Pcb. This is a test facility for the Trigger Pcb. This setting must not be selected when the Trigger Pcb is connected to the Modulator Pcb, as operating with no warm up delay is likely to damage the magnetron, or the Modulator.

USE OF FAULT FINDING FLOW CHARTS.

The flow charts are designed to locate faults to a module level.

They are broken down into fault categories rather than modules.

Where possible Entry Points are defined for particular faults.

| Fault | Sheet | Entry Point |
| --- | --- | --- |
| Transceiver not functioning at all. | 1 | A |
| Yellow Mains On indicator not illuminated or flashing. | 1 | A |
| Green Transceiver On indicator not illuminated. | 1 | A |
| BIST Voltages incorrect. *** | 1a | Z |
| Antenna not rotating. * | 2 | B |
| Heading Marker Fail error. *** | 3 | C |
| No Azimuth Pulses No Heading Marker. | 3 | D |
| No Trigger. | 4 | E |
| Charge Trigger fail BIST. *** | 4 | E |
| Modulator trigger fail BIST. *** | 4 | E |
| Transceiver not transmitting *** | 4 | F |
| Spark Gap Detect error. *** | 4 | G |
| Red Transmitting indicator not illuminated. | 5 | J |
| Transceiver fails to select Transmit. | 5 | J |
| No Video. | 5 | H |
| Receiver Faults. | 5a | K |
| Message Fail error. *** | 6 | L |
| Corrupt Data error. *** | 6 | L |
| No communication from Transceiver | 6 | M |
| No Pre-Trigger. | 7 | N |
| Transceiver fails to respond to External Trigger | 7 | P |
| Modulator HT incorrect BIST. *** | 8 | Q |
| Magnetron Current Incorrect BIST. *** | 8a | S |
| Transmit tellback error | 9 | T |
| Inhibit tellback error | 5b | U |
| Pulse Length tellback error | 9 | V |
| Wide-band tellback error | 9 | W |
| AFC On tellback error | 5 | X |
| Timer error | Change Trigger Pcb |  |

Notes:

*	Sheet 2 only applies to transceiver type 65825CA, when used in conjunction with a standard LMS “X” Band Turning Unit.

*** 	This data is available on the serial data link from the transceiver as part of BIST.