Procedure to eliminate `water mark' sometimes visible on the Monitor

1. Gain access to the CRT Drive PCB.

3. Retaining the solder tag, cut the spade terminal off leaving a length of 35mm of wire. Trim end and prepare for soldering

4. Identify D23 and prepare bottom end for soldering.

5. Solder lead to bottom end of D23 and fit solder tag end to left hand side of PLT