CHAPTER 2

SYSTEM WIRING SCHEDULE AND COMMISSIONING

CONTENTS

1	INSTALLATION	2

2	CABLE SCHEDULES	3

3	COMMISSIONING INTRODUCTION	17

4	DEFAULT SETTINGS	17

5	COMMISSIONING	26

Illustrations

FIGURE 2.1 Cabling Diagram	16

PART 1

## INSTALLATION AND WIRING

# 1	INSTALLATION

Installation details for the units that make up the system may be found in the following manuals, mounting details are also included with the equipment

despatch kits.

These manuals should be consulted before starting the installation

| Unit Type | Unit Type Number | ## Manual |  |  |
| --- | --- | --- | --- | --- |
|  |  | ## Number | Ref. |  |
| 25kW Transceiver | 65825CA | 65825010 |  | Chapter 2 |
| Display Unit | 65800SAV/1/165821R/MU | 65800010B |  | Chapter 3 |
| Scanner Control Unit* | 65427AH | NRA2524 |  | Chapter 3 |
| Isolating Switch | 65800700 | 65800012 |  | Chapter 6 |
| Transceiver Compatibility Unit | 65841F | 65800012 |  | Chapter 2 |
| Interswitch Unit | 65842F | 65800012 |  | Chapter 3 |
| Isolating Transformer | 96400750 | 65800012 |  | Chapter 6 |
| Video Control Unit | 65857A | 65857010 |  | Chapter 2 |

*Installation details are the same as for 65837AH.

# 2	CABLE SCHEDULES

| Cable No: 1 |  |  | Cable Type: HXXM(C) 3*2.5 |  |  |
| --- | --- | --- | --- | --- | --- |
| From: SCANNER CONTROL UNIT |  |  | To: SCOUT PROCESSOR UNIT |  |  |
| Unit Connector No: CONTACTOR-E/TAG |  |  | Unit Connector No: CD1 |  |  |
| Cable Connector Type: NONE |  |  | Cable Connector Type: D38999/26WE06BN |  |  |
| Line No. | Function | Pin No. | Colour | Pin No. | Remarks |
| 1 | AC LINE | TRIP6 | BK | A |  |
| 2 | AC NEUTRAL | TRIP2 | B | B |  |
| 3 | EARTH (0V) | E/TAG | BN | C |  |

| Cable No: 2 | Cable Type: 9556 752 386 |
| --- | --- |
| From: SCOUT PROCESSOR UNIT | To: TURNING UNIT |

| Cable No: 3 | Cable Type: 2HXXM(B) 5*2*0.4 |
| --- | --- |
| From: SCOUT PROCESSOR UNIT | To: BTS NAVRAD |

| Cable No: 4 |  |  | Cable Type: 2HXXM(B)5*2*0.4 |  |  |
| --- | --- | --- | --- | --- | --- |
| From: SCOUT PROCESSOR UNIT |  |  | To: PULSE TRANSCEIVER |  |  |
| Unit Connector No: CD7 |  |  | Unit Connector No: TSD |  |  |
| Cable Connector Type: D38999/26WA35BN |  |  | Cable Connector Type: 171643 BL5.08/13 |  |  |
| Line No. | Function | Pin No. | Colour | Pin No. | Remarks |
| 1 | ENABLE CONTACT | 2 | BK | TSD9 |  |
| 2 | ENABLE RETURN | 1 | W | TSD10 |  |
| 3 | SCREEN | CONE | BRAID | CONE |  |

| Cable No: 5 |  |  | Cable Type: 16-2-4C |  |  |
| --- | --- | --- | --- | --- | --- |
| From: TRANSCEIVER COMPATIBILITY UNIT |  |  | To: VIDEO CONTROL UNIT |  |  |
| Unit Connector No: TSC |  |  | Unit Connector No: TSD |  |  |
| Cable Connector Type: 171633 BL5.08/3 |  |  | Cable Connector Type: 171633 BL5.08/3 |  |  |
| Line No | Function | Pin No. | Colour | Pin No. | Remarks |
| 1 | +15V EXT | 1 | R | 1 |  |
| 2 | 0V EXT | 2 | G | 3 |  |
| 3 | 0V EXT | 2 | Y | 3 |  |
| 4 | -15V EXT | 3 | B | 5 |  |
| 5 | SCREEN | E/TAG | BRAID | E/TAG |  |

Cable No. 6 n/a

| Cable No: 7 |  |  | Cable Type: 6224C (BLACK) |  |  |
| --- | --- | --- | --- | --- | --- |
| From: TRANSCEIVER COMPATIBILITY UNIT |  |  | To: PULSE TRANSCEIVER |  |  |
| Unit Connector No: TSG |  |  | Unit Connector No: TSG |  |  |
| Cable Connector Type: 171634 BL5.08/4 |  |  | Cable Connector Type: 171636 BL5.08/6 |  |  |
| Line No. | Function | Pin No. | Colour | Pin No. | Remarks |
| 1 | AZI OUT+ | 1 | B/W | TSG1 |  |
| 2 | AZI OUT- | 2 | W/B | TSG2 |  |
| 3 | HMKR OUT+ | 3 | O/W | TSG3 |  |
| 4 | HMKR OUT- | 4 | W/O | TSG4 |  |
| 5 | SCREEN | E/TAG | BRAID | CONE |  |

| Cable No: 8 |  |  | Cable Type: 2HXXM(B) 5*2*0.4 |  |  |
| --- | --- | --- | --- | --- | --- |
| From: TRANSCEIVER COMPATIBILITY UNIT |  |  | To: SCOUT PROCESSOR UNIT |  |  |
| Unit Connector No: TSF |  |  | Unit Connector No: CD3 |  |  |
| Cable Connector Type: 171638 BL5.08/8 |  |  | Cable Connector Type: D38999/26WF35BN |  |  |
| Line No. | Function | Pin No. | Colour | Pin No. | Remarks |
| 1 | SCOUT DATA+ | 1 | BK | 37 | Twisted pair |
| 2 | SCOUT DATA- | 2 | W/BK | 46 |  |
| 3 | AZI IN+ | 3 | BK | 17 | Twisted pair |
| 4 | AZI IN- | 4 | W/R | 18 |  |
| 5 | HMKR IN+ | 5 | BK | 22 | Twisted pair |
| 6 | HMKR IN- | 6 | W | 30 |  |
| 7 | SCOUT ERROR | 7 | BK | 21 | Twisted pair |
| 8 | SCOUT ERROR RTN | 8 | W | 29 |  |
| 9 | SCREEN | E/TAG | BRAID | CONE |  |

| Cable No: 9 |  |  | Cable Type: EPD62590A |  |  |
| --- | --- | --- | --- | --- | --- |
| From: INTERSWITCH UNIT |  |  | To: BRIDGE DISPLAY UNIT |  |  |
| Unit Connector No: TSSC |  |  | Unit Connector No: TSD |  |  |
| Cable Connector Type: 171637 BL5.08//7 |  |  | Cable Connector Type: 171643 BL5.08/13 |  |  |
| Line No | Function | Pin No. | Colour | Pin No. | Remarks |
| 1 | +15V | 5 | R | 10 |  |
| 2 | 0V | 6 | B | 12 |  |
| 3 | SCREEN | E/TAG | BRAID | E/TAG |  |

| Cable No: 10 |  |  | Cable Type: RG59 B/U |  |  |
| --- | --- | --- | --- | --- | --- |
| From: TRANSCEIVER COMPATIBILITY UNIT |  |  | To: SCOUT PROCESSOR UNIT |  |  |
| Unit Connector No: SKZ |  |  | Unit Connector No: CD5 |  |  |
| Cable Connector Type: 11BNC-75-4-2 |  |  | Cable Connector Type: UG-603A/U |  |  |
| Line No. | Function | Pin No. | Colour | Pin No. | Remarks |
| 1 | SCOUT SYNC IN | PIN | INNER | PIN |  |
| 2 | SCREEN | OUTER | BRAID | OUTER |  |

| Cable No: 11 |  |  | Cable Type: EPD62590A |  |  |
| --- | --- | --- | --- | --- | --- |
| From: INTERSWITCH UNIT |  |  | To: CIC DISPLAY UNIT |  |  |
| Unit Connector No: TSSD |  |  | Unit Connector No: TSD |  |  |
| Cable Connector Type: 171637 BL5.08//7 |  |  | Cable Connector Type: 171643 BL5.08/13 |  |  |
| Line No | Function | Pin No. | Colour | Pin No. | Remarks |
| 1 | +15V | 5 | R | 10 |  |
| 2 | 0V | 6 | B | 12 |  |
| 3 | SCREEN | E/TAG | BRAID | E/TAG |  |

Cable No.12 n/a

| Cable No: 13 |  |  | Cable Type: RG59 B/U |  |  |
| --- | --- | --- | --- | --- | --- |
| From: TRANSCEIVER COMPATIBILITY UNIT |  |  | To: SCOUT PROCESSOR UNIT |  |  |
| Unit Connector No: SKY |  |  | Unit Connector No: CD6 |  |  |
| Cable Connector Type: 11BNC-75-4-2 |  |  | Cable Connector Type: UG-603A/U |  |  |
| Line No. | Function | Pin No. | Colour | Pin No. | Remarks |
| 1 | SCOUT VIDEO IN | PIN | INNER | PIN |  |
| 2 | SCREEN | OUTER | BRAID | OUTER |  |

| Cable No: 14 |  |  | Cable Type: 16-2-2C |  |  |
| --- | --- | --- | --- | --- | --- |
| From: TRANSCEIVER COMPATIBILITY UNIT |  |  | To: SCANNER CONTROL UNIT |  |  |
| Unit Connector No: TSD |  |  | Unit Connector: SA and TSA |  |  |
| Cable Connector Type: 171632 BL5.08/2 |  |  | Cable Connector Type: NONE REQUIRED |  |  |
| Line No. | Function | Pin No. | Colour | Pin No. | Remarks |
| 1 | +24V (EXT) | PIN | R | SA N |  |
| 2 | +24VRTN (EXT) | OUTER | B | TSA1 |  |
| 3 | SCREEN | E/TAG | BRAID | E/TAG |  |

| Cable No: 15 |  |  | Cable Type: 16-2-2C |  |  |
| --- | --- | --- | --- | --- | --- |
| From: TRANSCEIVER COMPATIBILITY UNIT |  |  | To ISOLATING SWITCH (1) |  |  |
| Unit Connector No: TSP |  |  | Unit Connector: SWITCH |  |  |
| Cable Connector Type: NONE REQUIRED |  |  | Cable Connector Type: NONE REQUIRED |  |  |
| Line No. | Function | Pin No. | Colour | Pin No. | Remarks |
| 1 | 230V AC LINE | 1 | R | 2 |  |
| 2 | 230V AC NEUTRAL | 4 | B | 4 |  |
| 3 | SCREEN | E/TAG | BRAID | E/TERM |  |

| Cable No: 16 |  |  | Cable Type: PT1YM |  |  |
| --- | --- | --- | --- | --- | --- |
| From: INTERSWITCH UNIT |  |  | To: VIDEO CONTROL UNIT |  |  |
| Unit Connector No: SKVC |  |  | Unit Connector No: INPUT VIDEO |  |  |
| Cable Connector Type : L734PNI |  |  | Cable Connector Type: 11BNC-75-4-2 |  |  |
| Line No. | Function | Pin No. | Colour | Pin No. | Remarks |
| 1 | VIDEO | PIN | INNER | PIN |  |
| 2 | SCREEN | OUTER | BRAID | OUTER |  |

| Cable No: 17 |  |  | Cable Type: PT1YM |  |  |
| --- | --- | --- | --- | --- | --- |
| From: TRANSCEIVER COMPATIBILITY UNIT |  |  | To: INTERSWITCH UNIT |  |  |
| Unit Connector No: SKV |  |  | Unit Connector: No: SKTA |  |  |
| Cable Connector Type : 11BNC-75-4-2 |  |  | Cable Connector: Type: L734PNI |  |  |
| Line No. | Function | Pin No. | Colour | Pin No. | Remarks |
| 1 | SCOUT VIDEO OUT | PIN | INNER | PIN |  |
| 2 | SCREEN | OUTER | BRAID | OUTER |  |

| Cable No: 18 |  |  | Cable Type: 6224C (BLACK) |  |  |
| --- | --- | --- | --- | --- | --- |
| From: TRANSCEIVER COMPATIBILITY UNIT |  |  | To: INTERSWITCH UNIT |  |  |
| Unit Connector No: TSE |  |  | Unit Connector: No TSTA |  |  |
| Cable Connector Type: 171636 BL5.08/6 |  |  | Cable Connector Type: 171638 BL5.08/8 |  |  |
| Line No. | Function | Pin No. | Colour | Pin No. | Remarks |
| 1 | DU DATA+ | 1 | B/W | 1 |  |
| 2 | DU DATA- | 2 | W/B | 2 |  |
| 3 | TX DATA+ | 3 | O/W | 3 |  |
| 4 | TX DATA- | 4 | W/O | 4 |  |
| 5 | TX TRIG+ | 5 | G/W | 5 |  |
| 6 | TX TRIG- | 6 | W/G | 6 |  |
| 7 | SCREEN | E/TAG | BRAID | E/TAG |  |

| Cable No: 19 |  |  | Cable Type: PT1YM |  |  |
| --- | --- | --- | --- | --- | --- |
| From INTERSWITCH UNIT |  |  | To: PULSE TRANSCEIVER |  |  |
| Unit Connector No: SKTB |  |  | Unit Connector: No SKW |  |  |
| Cable Connector Type: L734PNI |  |  | Cable Connector Type: 11BNC-75-4-2 |  |  |
| Line No. | Function | Pin No. | Colour | Pin No. | Remarks |
| 1 | VIDEO | PIN | INNER | PIN |  |
| 2 | SCREEN | OUTER | BRAID | OUTER |  |

| Cable No: 20 |  |  | Cable Type- 6224C (BLACK) |  |  |
| --- | --- | --- | --- | --- | --- |
| From: INTERSWITCH UNIT |  |  | To: PULSE TRANSCEIVER |  |  |
| Unit Connector No: TSTB |  |  | Unit Connector No: TSD |  |  |
| Cable Connector Type: 171638 BL5.08/8 |  |  | Cable Connector Type: 171643 BL5.08/13 |  |  |
| Line No. | Function | Pin No. | Colour | Pin No. | Remarks |
| 1 | DU DATA+ | 1 | B/W | 1 |  |
| 2 | DU DATA- | 2 | W/B | 2 |  |
| 3 | TX DATA+ | 3 | O/W | 3 |  |
| 4 | TX DATA- | 4 | W/O | 4 |  |
| 5 | TX TRIG+ | 5 | G/W | 5 |  |
| 6 | TX TRIG- | 6 | W/G | 6 |  |
| 7 | SCREEN | E/TAG | BRAID | CONE |  |

| Cable No: 21 |  |  | Cable Type: 6224C (BLACK) |  |  |
| --- | --- | --- | --- | --- | --- |
| From: INTERSWITCH UNIT |  |  | To: VIDEO CONTROL UNIT |  |  |
| Unit Connector No: TSDC |  |  | Unit Connector No: TSA |  |  |
| Cable Connector Type: 171638 BL5.08/8 |  |  | Cable Connector Type: 171638 BL5.08/8 |  |  |
| Line No. | Function | Pin No. | Colour | Pin No. | Remarks |
| 1 |  |  | B/W |  |  |
| 2 |  |  | W/B |  |  |
| 3 | TX DATA+ | 3 | O/W | 3 |  |
| 4 | TX DATA- | 4 | W/O | 4 |  |
| 5 | TX TRIG+ | 5 | G/W | 5 |  |
| 6 | TX TRIG- | 6 | W/G | 6 |  |
| 7 | SCREEN | E/TAG | BRAID | E/TAG |  |

Cable No.22 n/a

| Cable No: 23 |  |  | Cable Type: PT1YM |  |  |
| --- | --- | --- | --- | --- | --- |
| From INTERSWITCH UNIT |  |  | To: CIC DISPLAY UNIT |  |  |
| Unit Connector No: SKVB |  |  | Unit Connector No: SKV |  |  |
| Cable Connector Type: L734PNI |  |  | Cable Connector Type: L734PNI |  |  |
| Line No. | Function | Pin No. | Colour | Pin No. | Remarks |
| 1 | DISPLAY UNIT B VIDEO | PIN | INNER | PIN |  |
| 2 | SCREEN | OUTER | BRAID | OUTER |  |

| Cable No: 24 |  |  | Cable Type: 6224C (BLACK) |  |  |
| --- | --- | --- | --- | --- | --- |
| From: INTERSWITCH UNIT |  |  | To: CIC DISPLAY UNIT |  |  |
| Unit Connector No: TSDB |  |  | Unit Connector No: TSA |  |  |
| Cable Connector Type: 171638 BL5.08/8 |  |  | Cable Connector Type: 171638 BL5.08/8 |  |  |
| Line No. | Function | Pin No. | Colour | Pin No. | Remarks |
| 1 | DU DATA+ | 1 | B/W | 1 |  |
| 2 | DU DATA- | 2 | W/B | 2 |  |
| 3 | TX DATA+ | 3 | O/W | 3 |  |
| 4 | TX DATA- | 4 | W/O | 4 |  |
| 5 | TX TRIG+ | 5 | G/W | 5 |  |
| 6 | TX TRIG- | 6 | W/G | 6 |  |
| 7 | SCREEN | E/TAG | BRAID | E/TAG |  |

| Cable No: 25 |  |  | Cable Type: 6224C (BLACK) |  |  |
| --- | --- | --- | --- | --- | --- |
| From: INTERSWITCH UNIT |  |  | To: CIC DISPLAY UNIT |  |  |
| Unit Connector No: TSSB |  |  | Unit Connector No: TSB |  |  |
| Cable Connector Type: 171637 BL5.08/7 |  |  | Cable Connector Type: 171637 BL5.08/7 |  |  |
| Line No. | Function | Pin No. | Colour | Pin No. | Remarks |
| 1 | ISW REQ+ | 1 | B/W | 1 |  |
| 2 | ISW REQ- | 2 | W/B | 2 |  |
| 3 | ISW ACK+ | 3 | O/W | 3 |  |
| 4 | ISW ACK- | 4 | W/O | 4 |  |
| 5 | +12V | 5 | G/W | 5 |  |
| 6 | 0V | 6 | W/G | 6 |  |
| 7 | 0V | 6 | BN/W | 6 |  |
| 8 | -12V | 7 | W/BN | 7 |  |
| 9 | SCREEN | E/TAG | BRAID | E/TAG |  |

| Cable No: 26 |  |  | Cable Type: FILOTEX 298760 |  |  |
| --- | --- | --- | --- | --- | --- |
| From: PULSE TRANSCEIVER |  |  | To: SHIP’S EW SYSTEM |  |  |
| Unit Connector No: SKM (via adaptor cable) |  |  | Unit Connector No: TBA |  |  |
| Cable Connector Type: UG-536B/U |  |  | Cable Connector Type: TBA |  |  |
| Line No. | Function | Pin No. | Colour | Pin No. | Remarks |
| 1 | PRE TRIG 2 | PIN | INNER |  |  |
| 2 | SCREEN | OUTER | BRAID |  |  |

| Cable No: 27 |  |  | Cable Type: FILOTEX 298760 |  |  |
| --- | --- | --- | --- | --- | --- |
| From: VIDEO CONTROL UNIT |  |  | To: BTS NAVRAD |  |  |
| Unit Connector No: OUTPUT VIDEO (via adaptor cable) |  |  | Unit Connector: CD8 |  |  |
| Cable Connector Type :UG-536B/U |  |  | Cable Connector: D38999/TV06RGW-11-01-PNLC |  |  |
| Line No. | Function | Pin No. | Colour | Pin No. | Remarks |
| 1 | VIDEO | PIN | INNER |  |  |
| 2 | SCREEN | OUTER | BRAID |  |  |

| Cable No: 28 |  |  | Cable Type: FILOTEX 298760 |  |  |
| --- | --- | --- | --- | --- | --- |
| From: VIDEO CONTROL UNIT |  |  | To: BTS NAVRAD |  |  |
| Unit Connector No: OUTPUT TRIGGER (via adaptor cable) |  |  | Unit Connector No: CD7 |  |  |
| Cable Connector Type: UG-536B/U |  |  | Cable Connector Type: D38999/TV06RGW-11-01-PNLC |  |  |
| Line No. | Function | Pin No. | Colour | Pin No. | Remarks |
| 1 | TRIG O/P | PIN | INNER |  |  |
| 2 | SCREEN | OUTER | BRAID |  |  |

Cable No. 29 n/a

Cable No. 30 n/a

| Cable No: 31 |  |  | Cable Type: 37-3-2R |  |  |
| --- | --- | --- | --- | --- | --- |
| From: ISOLATING TRANSFORMER (1) |  |  | To: ISOLATING SWITCH (1) |  |  |
| Unit Connector No: TRANSFORMER |  |  | Unit Connector: SWITCH |  |  |
| Cable Connector Type: NONE REQUIRED |  |  | Cable Connector Type: NONE REQUIRED |  |  |
| Line No. | Function | Pin No. | Colour | Pin No. | Remarks |
| 1 | 230V AC LINE | 6 | R | 1 |  |
| 2 | 230V AC NEUTRAL | 7 | B | 3 |  |
| 3 | SCREEN | CONE | BRAID | E/TERM |  |

| Cable No: 32 |  |  | Cable Type: EPD62657A |  |  |
| --- | --- | --- | --- | --- | --- |
| From: ISOLATING SWITCH 1 |  |  | To: PULSE TRANSCEIVER |  |  |
| Unit Connector No: SWITCH |  |  | Unit Connector: PLA |  |  |
| Cable Connector Type: NONE REQUIRED |  |  | Cable Connector Type: KPSE 6 E 12-3SDZ & boot 222K 142-3/42-0 |  |  |
| Line No. | Function | Pin No. | Colour | Pin No. | Remarks |
| 1 | 230V AC LINE | 2 | BN | a |  |
| 2 | 230V AC NEUTRAL | 4 | B | b |  |
| 3 | EARTH | E/TERM | BRAID | CONE |  |

| Cable No: 33 |  |  | Cable Type: 16-2-2C |  |  |
| --- | --- | --- | --- | --- | --- |
| From: ISOLATING SWITCH 1 |  |  | To: VIDEO CONTROL UNIT |  |  |
| Unit Connector No: SWITCH |  |  | Unit Connector: TSP |  |  |
| Cable Connector Type: NONE REQUIRED |  |  | Cable Connector Type NONE REQUIRED |  |  |
| Line No. | Function | Pin No. | Colour | Pin No. | Remarks |
| 1 | 230V AC LINE | 2 | R | 1 |  |
| 2 | 230V AC NEUTRAL | 4 | B | 4 |  |
| 3 | SCREEN | E/TERM | BRAID | E/TAG |  |

| Cable No: 34 (SAGEM W8) |  |  | Cable Type: 16-2-2C |  |  |
| --- | --- | --- | --- | --- | --- |
| From: SHIP’S GPS |  |  | To: BRIDGE DISPLAY UNIT |  |  |
| Unit Connector No: |  |  | Unit Connector No: TSE |  |  |
| Cable Connector Type: 26FC98PA |  |  | Cable Connector Type: 171633 BL5.08/3 |  |  |
| Line No. | Function | Pin No. | Colour | Pin No. | Remarks |
| 1 | GPS+ | B | R | 1 |  |
| 2 | GPS- | A | B | 3 |  |
| 3 | SCREEN | E/TERM | BRAID | E/TAG |  |

| Cable No: 35 (SAGEM W10) |  |  | Cable Type: 16-2-6C |  |  |
| --- | --- | --- | --- | --- | --- |
| From: SHIP’S COMPASS |  |  | To: BRIDGE DISPLAY UNIT |  |  |
| Unit Connector No: |  |  | Unit Connector No: TSC |  |  |
| Cable Connector Type: 26FC98PB |  |  | Cable Connector Type: 171640 BL5.08/10 |  |  |
| Line No. | Function | Pin No. | Colour | Pin No. | Remarks |
| 1 | COMPASS S1 | A | R | 1 | 1:1 COMPASS |
| 2 | COMPASS S2 | B | B | 2 |  |
| 3 | COMPASS S3 | C | G | 3 |  |
| 4 | SYNCHRO REF | E | Y | 7 |  |
| 5 | REF RETURN | F | W | 10 |  |
| 6 |  |  | BK |  |  |
| 7 | SCREEN | TBA | BRAID | E/TAG |  |

| Cable No: 36 |  |  | Cable Type: 6224C (BLACK) |  |  |
| --- | --- | --- | --- | --- | --- |
| From: BRIDGE DISPLAY UNIT |  |  | To: INTERSWITCH UNIT |  |  |
| Unit Connector No: TSB |  |  | Unit Connector No: TSSA |  |  |
| Cable Connector Type: 171637 BL5.08/7 |  |  | Cable Connector Type: 171637 BL5.08/7 |  |  |
| Line No. | Function | Pin No. | Colour | Pin No. | Remarks |
| 1 | ISW REQ+ | 1 | B/W | 1 |  |
| 2 | ISW REQ - | 2 | W/B | 2 |  |
| 3 | ISW ACK+ | 3 | O/W | 3 |  |
| 4 | ISW ACK- | 4 | W/O | 4 |  |
| 5 | +12V | 5 | G/W | 5 |  |
| 6 | 0V | 6 | W/G | 6 |  |
| 7 | 0V | 6 | BN/W | 6 |  |
| 8 | -12V | 7 | W/BN | 7 |  |
| 9 | SCREEN | E/TAG | BRAID | E/TAG |  |

| Cable No: 37 |  |  | Cable Type: 6224C (BLACK) |  |  |
| --- | --- | --- | --- | --- | --- |
| From: BRIDGE DISPLAY UNIT |  |  | To: INTERSWITCH UNIT |  |  |
| Unit Connector No: TSA |  |  | Unit Connector No: TSDA |  |  |
| Cable Connector Type: 171638 BL5.08/8 |  |  | Cable Connector Type: 171638 BL5.08/8 |  |  |
| Line No. | Function | Pin No. | Colour | Pin No. | Remarks |
| 1 | DU DATA+ | 1 | B/W | 1 |  |
| 2 | DU DATA- | 2 | W/B | 2 |  |
| 3 | TX DATA+ | 3 | O/W | 3 |  |
| 4 | TX DATA- | 4 | W/O | 4 |  |
| 5 | TX TRIG+ | 5 | G/W | 5 |  |
| 6 | TX TRIG- | 6 | W/G | 6 |  |
| 7 | SCREEN | E/TAG | BRAID | E/TAG |  |

| Cable No: 38 |  |  | Cable Type: PT1YM |  |  |
| --- | --- | --- | --- | --- | --- |
| From: BRIDGE DISPLAY UNIT |  |  | To: INTERSWITCH UNIT |  |  |
| Unit Connector No: SKV |  |  | Unit Connector No: SKVA |  |  |
| Cable Connector Type: L734PNI |  |  | Cable Connector Type: L734PNI |  |  |
| Line No. | Function | Pin No. | Colour | Pin No. | Remarks |
| 1 | RADAR VIDEO IN | PIN | INNER | PIN |  |
| 2 | SCREEN | OUTER | BRAID | OUTER |  |

| Cable No: 39 (SAGEM W9) |  |  | Cable Type: 16-2-2C |  |  |
| --- | --- | --- | --- | --- | --- |
| From: SHIP’S LOG |  |  | To: BRIDGE DISPLAY UNIT |  |  |
| Unit Connector No: |  |  | Unit Connector No: TSF |  |  |
| Cable Connector Type: 26FC98PA |  |  | Cable Connector Type: 171633 BL5.08/3 |  |  |
| Line No. | Function | Pin No. | Colour | Pin No. | Remarks |
| 1 | LOG+ | B | R | 1 |  |
| 2 | LOG- | A | B | 3 |  |
| 3 | SCREEN | E/TERM | BRAID | E/TAG |  |

| Cable No: 40 |  |  | Cable Type: 37-3-2R |  |  |
| --- | --- | --- | --- | --- | --- |
| From: ISOLATING SWITCH (2) |  |  | To: BRIDGE DISPLAY UNIT |  |  |
| Unit Connector No: SWITCH |  |  | Unit Connector No: TSP |  |  |
| Cable Connector Type: NONE REQUIRED |  |  | Cable Connector Type: NONE REQUIRED |  |  |
| Line No. | Function | Pin No. | Colour | Pin No. | Remarks |
| 1 | 230 V AC LINE | 2 | R | 2 |  |
| 2 | 230V AC NEUTRAL | 4 | B | 4 |  |
| 3 | SCREEN | E/TERM | BRAID | E/TAG |  |

| Cable No: 41 |  |  | Cable Type: TBA |  |  |
| --- | --- | --- | --- | --- | --- |
| From: MAN ALOFT ISOLATING SWITCH |  |  | To: SHIP’S SWITCHFUSE |  |  |
| Unit Connector No: |  |  | Unit Connector No: |  |  |
| Cable Connector Type: NONE REQUIRED |  |  | Cable Connector Type: NONE REQUIRED |  |  |
| Line No. | Function | Pin No. | Colour | Pin No. | Remarks |
| 1 | AC LINE | 1 | TBA | TBA |  |
| 2 | AC NEUTRAL | 3 | TBA | TBA |  |
| 3 | EARTH | E/TERM | BRAID | TBA |  |

| Cable No: 42 |  |  | Cable Type: 37-3-2R |  |  |
| --- | --- | --- | --- | --- | --- |
| From: MAN ALOFT ISOLATING SWITCH |  |  | To: SCANNER CONTROL UNIT |  |  |
| Unit Connector No: |  |  | Unit Connector No: SA |  |  |
| Cable Connector Type: NONE REQUIRED |  |  | Cable Connector Type: NONE REQUIRED |  |  |
| Line No. | Function | Pin No. | Colour | Pin No. | Remarks |
| 1 | AC LINE | 2 | R | 6/T3 |  |
| 2 | AC NEUTRAL | 4 | B | 4T/2 |  |
| 3 | EARTH | E/TERM | BRAID | E/TAG |  |

| Cable No: 43 |  |  | Cable Type: 37-3-2R |  |  |
| --- | --- | --- | --- | --- | --- |
| From: MAN ALOFT ISOLATING SWITCH |  |  | To: ISOLATING TRANSFORMER (1) |  |  |
| Unit Connector No: |  |  | Unit Connector No: |  |  |
| Cable Connector Type: NONE REQUIRED |  |  | Cable Connector Type: NONE REQUIRED |  |  |
| Line No. | Function | Pin No. | Colour | Pin No. | Remarks |
| 1 | AC LINE | 2 | R | 1 | SEE NOTE |
| 2 | AC NEUTRAL | 4 | B | 4 |  |
| 3 | EARTH | E/TERM | BRAID | CONE |  |

FOR 115V INPUT LINK TERMINALS 1 TO 3 AND 2 TO 4 WITHIN ISOLATING TRANSFORMER

| Cable No: 44 |  |  | Cable Type: SHIPYARD SUPPLY |  |  |
| --- | --- | --- | --- | --- | --- |
| From: SHIP’S SWITCHFUSE |  |  | To: ISOLATING TRANSFORMER (2) |  |  |
| Unit Connector No: |  |  | Unit Connector No: |  |  |
| Cable Connector Type: NONE REQUIRED |  |  | Cable Connector Type: NONE REQUIRED |  |  |
| Line No. | Function | Pin No. | Colour | Pin No. | Remarks |
| 1 | 115V AC LINE | TBA | TBA | 1 | SEE NOTE |
| 2 | 115V AC NEUTRAL | TBA | TBA | 4 |  |
| 3 | EARTH | TBA | BRAID | CONE |  |

FOR 115V INPUT LINK TERMINALS 1 TO 3 AND 2 TO 4 WITHIN ISOLATING TRANSFORMER

| Cable No: 45 |  |  | Cable Type: 37-3-2R |  |  |
| --- | --- | --- | --- | --- | --- |
| From: ISOLATING TRANSFORMER (2) |  |  | To: ISOLATING SWITCH (2) |  |  |
| Unit Connector No: TRANSFORMER |  |  | Unit Connector: SWITCH |  |  |
| Cable Connector Type: NONE REQUIRED |  |  | Cable Connector Type: NONE REQUIRED |  |  |
| Line No. | Function | Pin No. | Colour | Pin No. | Remarks |
| 1 | 230V AC LINE | 6 | R | 1 |  |
| 2 | 230V AC NEUTRAL | 7 | B | 3 |  |
| 3 | SCREEN | CONE | BRAID | E/TERM |  |

| Cable No: 46 |  |  | Cable Type: 37-3-2R |  |  |
| --- | --- | --- | --- | --- | --- |
| From: ISOLATING TRANSFORMER (2) |  |  | To: ISOLATING SWITCH (3) |  |  |
| Unit Connector No: TRANSFORMER |  |  | Unit Connector: SWITCH |  |  |
| Cable Connector Type: NONE REQUIRED |  |  | Cable Connector Type: NONE REQUIRED |  |  |
| Line No. | Function | Pin No. | Colour | Pin No. | Remarks |
| 1 | 230V AC LINE | 6 | R | 1 |  |
| 2 | 230V AC NEUTRAL | 7 | B | 3 |  |
| 3 | SCREEN | CONE | BRAID | E/TERM |  |

| Cable No: 47 (SAGEM W11) |  |  | Cable Type: 16-2-2C |  |  |
| --- | --- | --- | --- | --- | --- |
| From: SHIP’S GPS |  |  | To: CIC DISPLAY UNIT |  |  |
| Unit Connector No: |  |  | Unit Connector No: TSE |  |  |
| Cable Connector Type: 26FC98PA |  |  | Cable Connector Type: 171633 BL5.08/3 |  |  |
| Line No. | Function | Pin No. | Colour | Pin No. | Remarks |
| 1 | GPS+ | B | R | 1 |  |
| 2 | GPS- | A | B | 3 |  |
| 3 | SCREEN | E/TERM | BRAID | E/TAG |  |

| Cable No: 48 (SAGEM W13) |  |  | Cable Type: 16-2-6C |  |  |
| --- | --- | --- | --- | --- | --- |
| From: SHIP’S COMPASS |  |  | To: CIC DISPLAY UNIT |  |  |
| Unit Connector No: |  |  | Unit Connector No: TSC |  |  |
| Cable Connector Type: 26FC98PB |  |  | Cable Connector Type: 171640 BL5.08/10 |  |  |
| Line No. | Function | Pin No. | Colour | Pin No. | Remarks |
| 1 | COMPASS S1 | A | R | 1 | 1:1 COMPASS |
| 2 | COMPASS S2 | B | B | 2 |  |
| 3 | COMPASS S3 | C | G | 3 |  |
| 4 | SYNCHRO REF | E | Y | 7 |  |
| 5 | REF RETURN | F | W | 10 |  |
| 6 |  |  | BK |  |  |
| 7 | SCREEN | TBA | BRAID | E/TAG |  |

| Cable No: 49 |  |  | Cable Type: HUCOM (TWCB)2*2*0.35 |  |  |
| --- | --- | --- | --- | --- | --- |
| From: BTS-NAVRAD |  |  | To: CIC DISPLAY UNIT |  |  |
| Unit Connector No: CD5 |  |  | Unit Connector No: SKY |  |  |
| Cable Connector Type: D38999/26WC35AN |  |  | Cable Connector Type: DB25P & MHDTZK25 SHELL |  |  |
| Line No. | Function | Pin No. | Colour | Pin No. | Remarks |
| 1 | TRACK DATA + | 21 |  | 24 |  |
| 2 | TRACK DATA - | 22 |  | 23 |  |
| 3 |  |  |  |  |  |
| 4 |  |  |  |  |  |
| 5 | SCREEN | CONE | BRAID | E/TAG |  |

| Cable No: 50 (SAGEM W12) |  |  | Cable Type: 16-2-2C |  |  |
| --- | --- | --- | --- | --- | --- |
| From: SHIP’S LOG |  |  | To: CIC DISPLAY UNIT |  |  |
| Unit Connector No: |  |  | Unit Connector No: TSF |  |  |
| Cable Connector Type: 26FC98PA |  |  | Cable Connector Type: 171633 BL5.08/3 |  |  |
| Line No. | Function | Pin No. | Colour | Pin No. | Remarks |
| 1 | LOG+ | B | R | 1 |  |
| 2 | LOG- | A | B | 3 |  |
| 3 | SCREEN | E/TERM | BRAID | E/TAG |  |

| Cable No: 51 |  |  | Cable Type: 37-3-2R |  |  |
| --- | --- | --- | --- | --- | --- |
| From: ISOLATING SWITCH (3) |  |  | To: CIC DISPLAY UNIT |  |  |
| Unit Connector No: SWITCH |  |  | Unit Connector No: TSP |  |  |
| Cable Connector Type: NONE REQUIRED |  |  | Cable Connector Type: NONE REQUIRED |  |  |
| Line No. | Function | Pin No. | Colour | Pin No. | Remarks |
| 1 | 230 V AC LINE | 2 | R | 2 |  |
| 2 | 230V AC NEUTRAL | 4 | B | 4 |  |
| 3 | SCREEN | E/TERM | BRAID | E/TAG |  |

FIGURE 2.1 Cabling Diagram

PART 2

COMMISSIONING

# 3	COMMISSIONING INTRODUCTION

This section defines the default settings and commissioning procedures for the Hellenic Navy Combattante III navigation radar system.

Standard commissioning procedures are covered in the individual unit manuals. Any special requirements are detailed in paragraph 6.

# 4	DEFAULT SETTINGS

This section lists the default settings for links and customising switches for the Navigation radar system. Unless otherwise stated these will be set as factory defaults and should not require resetting unless, a component part is changed, the function is an optional operational setting, or is a ship specific setting.

Transceiver 65825CA

Input PCB (65825820)

Azimuth Type:	RS422

| Switch 2 Setting |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Position | 1 | 2 | 3 | 4 | 5 | 6 |
| Setting | Off | Off | On | Off | Off | On |

Radar Silence (Transmit enable) Type:	Closing contact

| Switch 3 Setting |  |  |  |  |
| --- | --- | --- | --- | --- |
| Position | 1 | 2 | 3 | 4 |
| Setting | Off | On | On | Off |

Control Data		Normal

| Switch 1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Position | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 |
| Setting | Off | Off | Off | Off | On | On | On | On |

Link settings:

LK1	Not fitted			For service use only

LK2	Not fitted			For service use only

LK5	Set		1 to 2		Normal

Modulator PCB (65825802)

Link settings:

LK	Set 	2 to 3		Heater turndown enabled

Trigger PCB (65825801)

| Switch1 |  |  |
| --- | --- | --- |
| Position | Setting | Function |
| 1 | Off | Timer in use |
| 2 | Off | Heading marker in use |
| 3 | On | Radar silence normal |
| 4 | Off | Heater turndown enabled |
| 5 | On | Timer short |
| 6 |  | Not used |
| 7 | On | Baud rate X2 |
| 8 | On | Baud rate X4 |

Link settings:

LK1	Fitted			Test link

LK2	Set 	1 to 2		Pre trigger only when transmitting

LK3	Set 	1 to 2		Pre trigger only when transmitting

LK4	Set 	1 to 2		Pre trigger only when transmitting

LK5	Set 	2 to 3		Use external heading marker

LK6	Set 	2 to 3		Use external azimuth pulses

LK7	Not fitted		Test link

LK8	Set 	2 to 3		Wobbulation enabled

LK11	Set	1 to 2		External manual tune selected

LK12	Set	1 to 2		AFC manual tuning selection enabled

LK14	Not fitted		Never used

Default pulse repetition frequency		set to 3.3kHz by RV1

Prf limiting

Short pulse					set to 3.3kHz by RV2

Medium pulse				set to 1.65kHz by RV3

Long pulse					set to 825Hz by RV4

Default pre-trigger setting

Factory set to:

Lead magnetron output by 		5s (set by RV9)

Width						2s (set by RV10)

Video Buffer PCB (65825805)

Video amplitude (terminated in 75)	3.0V (set by RV1)

Polarity negative

Link settings:

LK1	Not fitted	75 output

LK2	Fitted		50 output. (this output is not used)

LK3	Fitted		50 output. (this output is not used)

LK4	Fitted		50 output  (this output is not used)

Display Processor Unit 65800SAV/1/1

The system configuration and initial set up is all accessed through the display menu system. See manual 65800010B Chapter 4. for the configuration procedure.

Items listed, as Ship dependant should be set as part of the commissioning procedure.

A complete checklist is included in the manual. This should be completed at commissioning, and all settings recorded.

The configuration settings for the Hellenic Navy Combattante III Navigation Radar system are listed below.

System Configuration (65800010B Chap. 4 para.3)

Interswitch	65842  2-WAY

BSH Mode	No

Distance Units	n.m. (nautical miles)

Remote Alarms	No

Ship’s Length	Ship dependent

Beam	Ship dependent

Max Rate of Turn	Ship dependent

STD Rate of Turn	Ship dependent

Nav Sensor Offset	Ship dependent

I/O Options (65800010B Chap. 4 para.4)

Serial Port 1 (Nav Data Input)

Baud Rate	4800 Baud (non adjustable)

Input Device	NAV SENSOR

Message Header	GLL, ZDA

Output Type	Don’t care

Note it is not necessary to set up an Input Type

Serial Port 2 (Ship’s Log Input)

Baud Rate	4800 Baud (non adjustable)

Input Device	LOG

Message Header	VHW

Output Type	Don’t care

Note it is not necessary to set up an Input Type

Radar Port (Track Table output)

Baud Rate	4800 Baud (non adjustable)

Output Device (for Bridge display)	NO OUTPUT

Output device (for CIC display)	TRACK TABLE TU REF or

TRACK TABLE CENTRE REF

Set at commissioning

Message Header	No setting required

Output Type	RS422

Analogue I/P (Compass)

Compass Fitted	Special (this is an indication only, and does not require setting)

Compass Type	SYNCHRO

Compass Ratio	1 : 1

Transceiver Settings (65800010B Chap. 4 para.5)

Cable Length	Ship dependent

Beamwidth	1.3°

Display Marker	HL (Heading Line)

TU Offsets	Ship dependent

TXRX Location	BLKHD

RF Feeder Length	Ship dependent

Sector Blanking	Ship dependent

Note:  In this system the SCOUT Transceiver is designated as TXA, the NAV Transceiver is TXB.

Sector blanking is not applied to the SCOUT Transceiver.

For TXA the sector should be set to be slightly larger than that defined within the SCOUT Processor.

Interswitch Configuration (65800010B Chap. 4 para.5)

This function does not apply to this system: no setting is required.

Initialisation in Transmit Mode (65800010B Chap. 4 para.8)

Follow the procedure detailed in the manual.

Note: The COARSE TUNING section does not apply to TXA (SCOUT)

The section dealing with the Performance Monitor does not apply to this system.

Before setting the VIDEO I/P level the Video normalisation procedure detailed in Chap. 2 para. 6 should be completed.

Transceiver Compatibility Unit 65841F (NRA2524 Chap. 3)

Power Supply PCB. (65841803)

LKA	Set 2 to 3	Normal operation

LK1	Fitted

LK2	Fitted		230V mains input

LK3	Not Fitted

LK4L	Fitted		Not used in this system

LK5L	Fitted		Not used in this system

LK4H	Not fitted	Not used in this system

LK5H	Not fitted 	Not used in this system

TCU Logic PCB. (65841802)

LK1	Set 2 to 3	75 output.

LK2	Set 2 to 3	75 output

LK3	Not Fitted

LK4	Set 1 to 2	Normal

LK5	Set 1 to 2	Normal

Video Outputs

SCOUT Video Out at SKV	3V into 75 (set by RV1)

AUX Video Out at SKX		Not used in this system

Interswitch Unit 65842F (NRA2524 Chap.3)

| Switch1 |  |  |
| --- | --- | --- |
| Position | Setting | Function |
| 1 | Off | DUA fitted |
| 2 | Off | DUB fitted |
| 3 | Off | DUC fitted |
| 4 | On | DUD not fitted |

| Switch 2 |  |  |
| --- | --- | --- |
| Position | Setting | Function |
| 1 | Off | TXA fitted (SCOUT) |
| 2 | Off | TXB fitted (NAV) |
| 3 | On | Not used |
| 4 | On | Not used |

LK1	Set 1 to 2	Local Mode.

LK2	Set 1 to 2	Normal.

Scanner Control Unit 65427AH (NRA2524 Chap.3)

Thermal Current Trip setting	Set to minimum i.e. fully ACW

Isolating Transformer 9640750 (65800012 Chap.6)

Primary tap set for 115V input

Video Control Unit 65857A (65857010 Chap. 2)

The VCU should be set as follows and the settings recorded on the Customising Record Sheets in the VCU manual 65857010 Appendix A.  Video level setting should be made after the video level normalization has been made in the Commissioning section.

Power Supply 65841803 - Link Settings

| Link | Function | Options | Setting |
| --- | --- | --- | --- |
| A | Start Test Link | Remote/Normal | 1-2 |
| 1 | High Voltage Isolation Link | Used for factory test only. | Fitted |
| 2 | Input voltage select | 230v Input | Fitted |
| 3 | Input voltage select | 115v Input | Not Fitted |
| 4L/4H | Motor Supply | Not used by the VCU | Not Fitted |
| 5L/5H | Motor Supply | Not used by the VCU | Not Fitted |

VCU Logic PCB 65857801- Link Settings

| Link | Function | Options | Setting |
| --- | --- | --- | --- |
| 1 | ATE Link | Always fitted | In |
| 2 | Test Link | Used for factory test only. | Out |
| 3 | TXDATA Termination | High or low impedance | 2-3 |
| 4 | RS422 Trigger Termination | High or low impedance | 2-3 |
| 5 | SART Termination | Not used | 2-3 |
| 6 | Cable compensation | Normal, short, long | 3-4 |
| 7 | Trigger Input Selection | Coaxial, RS422,delayed O/P | 3-4 |
| 8 | MIS Cable Termination | 50, 75, high impedance | 1-2 |
| 9 | Trigger I/P Cable Termination | 50, 75, high impedance | 1-2 |
| 10 | Video I/P Cable Termination | 50, 75, high impedance | 1-2 |
| 11 | Video I/P Polarity | Positive or negative | 2-3 |
| 12 | Video Bypass | Bypass, bypass special, no bypass | Parked pin 3 |
| 13 | Trigger O/P Source Impedance | 10,39, 49 | 3-4 |
| 14 | Trigger Bypass | Bypass, bypass special, no bypass | Parked pin 2 |
| 15 | Video O/P Source impedance | 30, 50, 75 | 1-2 |
| 16 | Video O/P Polarity | Positive or negative | 1-2 |

VCU Logic PCB 65857801- Switch Settings

S1 — Spike

These switches set the depth of clutter suppression at the centre of the picture.

Start with the default values shown below, then adjust for optimum performance.

| S1-1 | S1- 2 |  |
| --- | --- | --- |
| OFF | OFF | Max |
| ON | OFF | Default |
| OFF | ON |  |
| ON | ON | Min |

Note: S1 - 3 and S1 - 4 are not used.

S2— AC Law

These switches set the radar range over which the sea clutter control operates when in MANUAL mode of operation. The setting needs to be adjusted according to the antenna height.

| S2-1 | S2- 2 | S2 - 3 | Antenna height above sea level (m) |
| --- | --- | --- | --- |
| OFF | OFF | OFF | 3 and below |
| ON | OFF | OFF | 4 - 11 |
| OFF | ON | OFF | 12 - 15 |
| ON | ON | OFF | 16 - 19 |
| OFF | OFF | ON | 20 - 23 |
| ON | OFF | ON | 24 - 27 |
| OFF | ON | ON | 28 - 31 |
| ON | ON | ON | 32 and above |

S3 — Setting DATA

These switches set system parameters

S3 - 1, S3 - 2 Transceiver type

These switches allow the video processing characteristic to be optimised for the type of transceiver in use.

| S3-1 | S3- 2 | Transceiver Type |
| --- | --- | --- |
| OFF | OFF | Non-Decca |
| ON | OFF | Series 90 |
| OFF | ON | BM1 and BM2 |
| ON | ON | BME (Default) |

S3 - 3, S3 - 4 Reserved for future use

S3 - 5, S3 - 6 Turn down setting

These switches set the maximum amount of clutter suppression that is allowed; if the setting is too low insufficient rain and sea clutter will be removed, if the setting is too high dark holes will appear in the video and targets may be lost.

It should not be necessary to change the default settings.

| S3-5 | S3-6 | Level |
| --- | --- | --- |
| OFF | OFF | -1 |
| ON | OFF | 0 (Default) |
| OFF | ON | +1 |
| ON | ON | +2 |

S3 - 7, S3 - 8 Initial Gain

In this system,  this function is disabled by setting S3-8 to ‘Disabled’. S3-7 is then non-effective.

| S3-7 | Initial Video Level |
| --- | --- |
| ON | >3V peak |
| OFF | <3V peak |

| S3-8 | AGC |
| --- | --- |
| ON | Disabled (normal) |
| OFF | Enabled (special) |

S4 Video Sampling

Dependant on antenna height and waveguide length it may be necessary to delay the time at which the video is sampled by the auto-clutter circuits. This is necessary to compensate for the hole that appears in the video return due to the slant range from the antenna to the surface of the sea and the delay in the signal traversing the wave guide.

Excessive clutter or a hole in the center of the picture is an indication that S4 may require adjustment.

There are 16 possible values of delay selectable by S4. The values follow a binary sequence where S4-1 is the LSB and S4-4 is the MSB.

Suggested initial settings are:

| Antenna Height metres | S4-1 | S4-2 | S4-3 | S4-4 | Switch Setting |
| --- | --- | --- | --- | --- | --- |
| 0-5 | Off | Off | Off | Off | 0 |
| 6-10 | On | Off | Off | Off | 1 |
| 11-15 | Of | On | Off | Off | 2 |
| 16-20 | On | On | Off | Off | 3 |
| 21-25 | Off | Off | On | Off | 4 |
| 26-30 | On | Off | On | Off | 5 |
| 31-35 | Off | On | On | Off | 6 |
| 36-40 | On | On | On | Off | 7 |
| 41-45 | Off | Off | Off | On | 8 |
| 46-50 | On | Off | Off | On | 9 |
| 51-55 | Off | On | Off | On | 10 |
| 56-60 | On | On | Off | On | 11 |
| 61-65 | Off | Off | On | On | 12 |
| 66-70 | On | Off | On | On | 13 |
| 71-75 | Off | On | On | On | 14 |
| 76-80 | On | On | On | On | 15 |

The above are only initial settings for BridgeMaster E radars with aloft transceivers. The switches should be adjusted for optimum performance with the radar switched to short pulse.

For bulkhead transceivers the switch settings in the above table should be increased by one increment for each 10 metres of waveguide used. E.g for an antenna height of 10m with a 10m length of waveguide the switch setting would be increased from 1 to 2

Settings for non BridgeMaster E radars may differ considerably, and should be found empirically starting from the above settings.

Presets

RV1 — Delayed Trigger Delay

This potentiometer is not operative in this system configuration

RV3 — Interference Rejection Level

This potentiometer is factory set and should not be adjusted.

RV4 — Video Input Level Set

This potentiometer sets the video input level to the video processing circuits. Refer to Commissioning section, Video Levels.

RV2 — Video Output Level Set

This potentiometer sets the video output level. Refer to Commissioning section, Video Levels.

# 5	COMMISSIONING

Video Levels

The video levels between the SCOUT and NAV radars need to be normalised before setting the video levels in the display units.

Setting Video Levels SCOUT and NAV Radars.

- Switch the system on, and at the display unit select MASTER SCOUT, TRANSMIT, BOOST. Confirm that the radar is correctly tuned.

- Using an Oscilloscope and an in-line coupler, monitor the video signal at SKVC in the Interswitch

- In the Transceiver Compatibility Unit,  adjust RV1 to set the peak level to be 3V.

- At the display unit select MASTER NAV, TRANSMIT, LP.

- Confirm that the peak level of video signal at SKVC is 3V. If it is not, adjust RV1 in the BridgeMaster E Transceiver to set the level to 3V. (For details see manual 65825010 Chap.6 Fig. 6.14 and 6.15)

Setting Video Levels – Video Control Unit (65857A)

Video Input Level

Select MASTER SCOUT, TRANSMIT, BOOST.

Use an oscilloscope to monitor TP21 on VCU Logic PCB 65857801. Adjust RV4 to set the peak amplitude of the video at TP21 to 3V.

Video Output Level

Select RAW mode of operation, and set the gain control fully clockwise.

Use an oscilloscope to monitor the video either at the Video Output BNC connector (or at LK15 pin 4 on VCU Logic PCB 65857801), adjust RV2 to set the video level to 2.75V.

Trigger Delay

The Trigger delays for the SCOUT and NAV radars need to be normalised.

- At the display select MASTER NAV, TRANSMIT, SP.

- Follow the procedure in manual 65800010B Chap. 4 par. 8.4.2 (TRIG DELAY)

- At the display select MASTER SCOUT, TRANSMIT, LOW.

- Within the SCOUT processor adjust the Trigger delay so that known features are displayed at the correct range. Ensure echoes from straight quaysides appear straight and there is no evidence of pushing or pulling.

Commissioning Procedures Location of Reference Information.

| Unit Type | Unit Type Number | Manual |  |  |
| --- | --- | --- | --- | --- |
|  |  | Number | Ref. |  |
| 25kW Transceiver | 65825CA | 65825010 |  | Chapter 2 |
| Display Unit | 65800SAV/1/1 65821R/MU | 65800010B |  | Chapter 3 |
| Scanner Control Unit* | 65427AH | NRA2542 |  | Chapter 3 |
| Isolating Switch | 65800700 | 65800012 |  | Chapter 6 |
| Transceiver Compatibility Unit | 65841F | NRA2542 |  | Chapter 3 |
| Interswitch Unit | 65842F | NRA2542 |  | Chapter 3 |
| Isolating Transformer | 9600750 | 65800012 |  | Chapter 6 |
| Video Control Unit | 65857A | 65857010 |  | Chapter 2 |

*Installation details are the same as for 65837AH.