CHAPTER 3

ADDENDA FOR NON-STANDARD UNITS

CONTENTS

GENERAL INTRODUCTION	2

SECTION 1: TRANSCEIVER COMPATIBILITY UNIT 65841F	3

SECTION 2: INTERSWITCH UNIT 65842F	14

SECTION 3: SCANNER CONTROL UNIT 65427AH	26

# GENERAL INTRODUCTION

A number of units used in the system are special variants of standard units. This section of the manual explains the differences and lists any non standard spares

# SECTION 1: TRANSCEIVER COMPATIBILITY UNIT 65841F

Contents

GENERAL DESCRIPTION.	4

INSTALLATION AND COMMISSIONING	5

TECHNICAL SPECIFICATION	10

ENVIRONMENTAL SPECIFICATION	13

SPARES LIST	13

Illustrations

FIGURE 3.1 TCU Mains Input Connections to TSP	6

FIGURE 3.2 Transceiver Compatibility Unit PSU PCB 65841803	7

FIGURE 3.3 Transceiver Compatibility Unit Logic PCB 65841802	8

FIGURE 3.4 Transceiver Compatibility Unit Installation Details	9

GENERAL DESCRIPTION.

The Transceiver Compatibility Unit processes the SCOUT azimuth, heading marker, sync, video and error signals into a form compatible with BridgeMaster E.

The Transceiver Compatibility unit is a bulkhead-mounted unit that comprises a Power Supply PCB with filtered mains input and a Logic PCB.

All input connections, with the exception of the mains, are made to the logic PCB.

While mains is present at the power supply, it is in a standby condition, on detecting the serial control message from BridgeMaster E it switches to its operating state. It is the presence of volts on the serial control message input that starts the power supply, not the message content.

The azimuth, heading marker and error signal from SCOUT are converted into a serial message to be transmitted to the BridgeMaster E system in RS422 format.

The azimuth and heading marker are also buffered and converted to RS422 format for use by the BridgeMaster E Transceiver.

The sync from the SCOUT Processor is converted from single ended coaxial form into RS422.

The video is buffered and provision is made to adjust the video amplitude to normalise the video between the SCOUT and BridgeMaster-E Transceivers.

The serial control message from BridgeMaster-E is buffered and re-transmitted to the SCOUT transceiver.

A video inverter is included to invert the negative video from the Interswitch into the positive signal required by the Video Control Unit and the BTS NAVRAD.

There are no operator controls associated with the unit but a commissioning procedure is carried out on installation to set up internal links.

INSTALLATION AND COMMISSIONING

Link Settings

The link settings on the Power Supply PCB are intended for testing and fault-finding purposes and are factory set for normal operation. They should only be moved/removed by a qualified Service Engineer.

The Logic PCB has two links that must be set on installation, LK1 and LK2, which define the output impedance of the video buffers. LK3 is a test link and is not fitted.

Power Supply PCB

LKA - Start Test Link

Normal Operation	Fit 2 to 3

Test Operation	Fit 1 to 2

LK1 - High Voltage Isolation Link (not used in this system)

Normal Operation	Fitted

Test Operation	Not Fitted

LK2 & LK3 - Input Voltage Select Links

230V	LK2 Fitted

115V	LK3 Fitted

LK4L & H, LK5L & H - Motor Supply Select Links (not used in this system)

Low Speed X-Band	LK4L & LK5L Fitted

High Speed X-Band	LK4H & LK5H Fitted

TCU Logic PCB

LK1 Aux Video OUT Impedance

50 Output	Fit 1 to 2

75 Output	Fit 2 to 3

LK2 SCOUT Video OUT Impedance

50 Output	Fit 1 to 2

75 Output	Fit 2 to 3

LK3 - ATE Test Link

Factory Setting	Not Fitted

LK4 Bypass Video Buffer

Normal	Fit 1 to 2

Bypass	Fit 2 to 3

LK5 Bypass Video Buffer

Normal	Fit 1 to 2

Bypass	Fit 2 to 3

FIGURE 3.1 TCU Mains Input Connections to TSP

FIGURE 3.2 Transceiver Compatibility Unit PSU PCB 65841803

FIGURE 3.3 Transceiver Compatibility Unit Logic PCB 65841802

FIGURE 3.4 Transceiver Compatibility Unit Installation Details

TECHNICAL SPECIFICATION

Inputs to Logic PCB

SCOUT Video Input

Amplitude	2.5V min to 4.5V max

Polarity	Negative

Impedance	75

Connection Type	BNC

SCOUT Sync Input

Amplitude	13.5V min to 16.5V max

Polarity	Positive

Duration	1s 10%

PRF	1000Hz

Impedance	75

Timing	0 to  -100ns w.r.t. T0

Connection Type	BNC

Auxiliary Video Input

Amplitude	2.5V min to 4.5V max

Polarity	Negative

Impedance	75

Connection Type	BNC

SCOUT Azimuth Input

Amplitude	RS422

Pulses per Revolution	4096

Input Impedance	100 AC coupled

Connector Type	Terminal block

SCOUT Heading Marker Input

Amplitude	RS422

Pulse Width	1.2ms at 24 RPM

Input Impedance	100 AC coupled

Active State	Space (binary 0)

Triggering	Leading edge (negative going)

Connector Type	Terminal block

SCOUT Error Signal

Amplitude	+5V 10%

Sense+5V 	denotes error

Floating denotes no error

Input Impedance 	1k for input voltages < 5V

Connector Type	Terminal block

Outputs from Logic PCB

Radar Video Output

Amplitude (adjustable)	<2.5v to >4.5V

Polarity	Negative

Impedance	75

50 selectable

Connector Type	BNC

Radar Trigger

Amplitude	RS422

Timing	As SCOUT trigger input

Duration	As SCOUT trigger input

Connector Type	BNC

Auxiliary Video Output

Amplitude (adjustable)	<2.5v to >4.5V

Polarity	Positive

Impedance	75

50 selectable

Connector Type	BNC

RS422 Buffered SCOUT Azimuth Data.

Azimuth Out

Amplitude	RS422

Pulse per Revolution	4096

Connector Type	Terminal block

Heading Marker

Amplitude	RS422

Pulse Width	1.2ms at 24 RPM

Active State	Space (binary 0)

Triggering	Leading edge (negative going)

Connector Type	Terminal block

Data to Display (TX DATA)

Amplitude	RS422

Baud Rate	76.8 kB

Connector Type	Terminal block

Control Data from Display (DUDATA)

Amplitude	RS422

Baud Rate	76.8kB

Connector Type 	Terminal block

Control Data to SCOUT

Amplitude 	RS422

Baud Rate	76.8kB

Connector Type	Terminal block

Power Supplies.

Inputs:

Single Phase AC input

230V (selectable by link)	230V  10% (factory setting)

115V (selectable by link)	115V 10%

Frequency	47Hz min	64Hz max

Over-voltage Transients	40% above nominal for 1 sec max without causing damage.

At or above this level, the input fuse may blow.

Pulsed transients	+ 1200V peak, rise time 2 to 10us, duration <20us. Common or differential mode.

Under-Voltage	Mains dropout is monitored and may cause the system to reset.

Outputs:

The output supplies are listed for information only.

Modulator HT.Voltage	140V ± 10%.

Current	10kW	600mA max.

25kW	800mA max.

+15V for Bite & Bearing	+15V ± 6% at 400mA max

+15V	+15V ± 10% at 1.5A max

-15V	-15V ± 10% at 100mA max

+24V	+24V ± 20% at 2A mean (4.6A peak)

Internal Circuit Power Supplies.

+15V	+15V ± 5%. at 100mA max

+5V	+5V ± 5% at 250mA max

-15V	-15V ± 5%. at 50mA max

+36V	+36V ± 5% at 50mA max

ENVIRONMENTAL SPECIFICATION

Temperature

Operating	-15°C to +55°C

Storage	-25°C to +70°C

Relative Humidity

95% at 40°C ( non condensing)

Compass Safe Distances

| Standard | Steering |
| --- | --- |
| 0.9m | 0.6m |

SPARES LIST

| Description | Part Number |
| --- | --- |
| Power Supply PCB | 65841803 |
| Logic PCB | 65841802 |
| Mains Filter | 1586513 |
| Fuse 3.1A | MA00007245 |

# SECTION 2: INTERSWITCH UNIT 65842F

Contents

GENERAL INTRODUCTION	15

INSTALLATION AND COMMISSIONING	17

INTERFACE SPECIFICATIONS	20

SYSTEM STATUS	21

INTERCONNECTIONS	21

TECHNICAL SPECIFICATION	24

ENVIRONMENTAL SPECIFICATION	24

SPARES LIST	25

Illustrations

FIGURE 3.5 Interswitched System	16

FIGURE 3.6 Interswitch Unit Installation Details	18

FIGURE 3.7  Interswitch Logic Pcb	19

FIGURE 3.8 Typical Example of Status Panel Readout	21

FIGURE 3.9 Block Diagram – Interswitched System	23

GENERAL INTRODUCTION

The Interswitch unit performs the necessary selection of data from either the SCOUT transceiver or the BridgeMaster E transceiver and distributes it to the two BridgeMaster E ARPA displays and via the Display Compatibility unit and the Video Control unit to the BTS NAVRAD. See FIGURE 3.5

The Interswitch similarly selects control data from one of the ARPA displays and routes it to the transceivers.

The Interswitch in combination with the control software in the ARPA displays ensures that only one of the ARPA displays is selected to control the active transceiver, and that the data from that transceiver is distributed to all three displays.

The display that is selected to be in control is defined as the Master; the others are defined as Slaves.

Either BridgeMaster E display can be selected to be Master.

There must always be one display selected as Master.

The Interswitch unit is bulkhead mounted, and comprises a purpose designed enclosure and one logic PCB.

Power to the unit is supplied from the two ARPA displays controlling it.

Therefore if either display is turned on power is present at the Interswitch unit.

All connections to the unit are made on the logic PCB.

The braids of interconnecting cables are clamped at the entry to the unit as part of the EMC screening.

Bypass connectors are provided to enable the Interswitch control logic to be manually bypassed in the event of a failure.

The operator has the following configuration options.

| Current Configuration | Alternative Available Configuration |
| --- | --- |
| Master of SCOUT | Master of BridgeMaster E |
| Master of BridgeMaster E | Master of SCOUT |
| Slave to BridgeMaster E | Master of BridgeMaster E |
| Slave to SCOUT | Master of SCOUT |

SCOUT

Transceiver

(TXA)

BridgeMaster E

Transceiver

(TXB)

Interswitch

CIC Display

DUB

Video Control Unit

FIGURE 3.5 Interswitched System

INSTALLATION AND COMMISSIONING

DIL SWITCHES

Two groups of DIL switches are used to indicate which Displays and Transceivers are connected to the Interswitch.

To indicate that a Display or Transceiver is connected to the Interswitch, set the appropriate switches to the off position.

SW1 selects Display units SW2 selects transceivers.

| Switch1 |  |  |
| --- | --- | --- |
| Position | Setting | Function |
| 1 | Off | DUA fitted |
| 2 | Off | DUB fitted |
| 3 | Off | DUC fitted |
| 4 | On | DUD not fitted |

| Switch 2 |  |  |
| --- | --- | --- |
| Position | Setting | Function |
| 1 | Off | TXA fitted (SCOUT) |
| 2 | Off | TXB fitted (NAV) |
| 3 | On | Not used |
| 4 | On | Not used |

Link Settings

LK1 Interswitch mode

LK1	Set 1 to 2	Local Mode.

LKI	Set 2 to 3	Global Mode

Link1 selects LOCAL only LOCAL mode is used for this system. This link is not active for this system.

LK2 Reset Defaults

LK2	Set 1 to 2	Normal.

LK2 	Set 2 – 3	Reset defaults

This would normally reset default configuration at power up. This link is not active for this system as the system automatically resets defaults at power up.

The system defaults to DUA (Bridge Display) master of TXA (SCOUT)

FIGURE 3.6 Interswitch Unit Installation Details

FIGURE 3.7 Interswitch Logic PCB

INTERFACE SPECIFICATIONS

Interface to Transceiver

There is a bi-directional serial port to send control signals to and to receive signals from the transceiver. This is an RS422 differential signal.

Trigger is an RS422 signal on a separate twisted pair,

Video is sent down a separate coaxial cable. The video is received via a coaxial cable.

Radar Video Input

Peak amplitude	-2.5V to -5.5V

Shoulder noise	-0.25V minimum

Input Impedance 	75

Interface to Display Unit  (Transceiver Information)

There is a bi-directional serial port to send signals to, and to receive signals from the display or compatibility unit. These are RS422 differential signals. Trigger is an RS422 signal on a separate twisted pair.

Video is sent down a separate coaxial cable.

Radar Video Output

Peak amplitude	-2.5V to -5.5V

Shoulder noise	-0.25V minimum

Input Impedance 	75

Control

There is a bi-directional serial port to send and receive signals from the display. These are RS422 differential signals. One receives the configuration requests from the Display, the second one returns acknowledge and configuration signals back to the Display.

Additional cables bring in power from the displays to power the unit.

Bypass Connections

On the Interswitch PCB, additional connectors are provided. These allow a display to be connected to a Transceiver, bypassing the Interswitch. In the standard Interswitch, this permits TXA to be connected to one Display and TXB to be connected to another of the four possible Displays. Two displays will therefore be out of action.

Note:

In this system the SCOUT Transceiver must be connected by the bypass connectors to the Display. The system requires the azimuth data from the SCOUT Scanner unit to work.

The connector for the BridgeMaster Transceiver should be disconnected.

The operator must select the mode of operation dependent on the transceiver connected.

i.e. Master of NAV if the BridgeMaster E transceiver is connected,

or Master of SCOUT if the SCOUT transceiver is connected.

Note:	MIST will not be available to any Display in Bypass Mode.

General Distribution Connectors (not used in this system)

These enable LOG, COMPASS, NAV I/P etc to be distributed to all the display

units via parallel connected terminal blocks. It comprises:

1 input terminal block and 4 output terminal blocks on the Interswitch PCB.

SYSTEM STATUS

Visual indication of the system status is provided.

Green and Red LEDs show whether a display is connected to Transceiver A or

Transceiver B.

Additional Green and Red LEDs show whether the Display Unit is master or slave to the Transceiver. If a Display has not been configured, then the associated LED will not be illuminated.

FIGURE 3.8 Typical Example of Status Panel Readout

Mutual Interference Suppression Trigger (MIST is not used in this system)

There is one input from an external source that is mixed with the triggers from each Transceiver. The combined signal is fed out as an external display MIST. Each Display is supplied with this composite trigger excluding the trigger from the Transceiver that the Display is currently connected to. The MIST trigger blanks the display video for about 12s following a trigger from a transceiver that is not selected by the display. This reduces interference from an adjacent radar on own ship.

INTERCONNECTIONS

All inputs and outputs enter and leave on the underside of the Interswitch Unit. All connections physically plug directly into the Interswitch logic PCB.

The cables to the Turning Units/Transceivers consist of a quad twisted pair cables with overall screen for Trigger, and the bi-directional serial data, and a separate coaxial  cable for the Video.

The cables to the Display Units consist of two quad twisted pair cables. One provides the Transceiver signals for Trigger, and bi-directional serial data, the other the bi-directional Interswitch control data, positive and negative power. The Transceiver video input and output are via co-ax cables, as is the output MIST.

The Interswitch Unit has one External Transceiver MIST co-ax input, one External Display MIST co-ax output and four co-ax Display MIST outputs.

FIGURE 3.9 Block Diagram – Interswitched System

TECHNICAL SPECIFICATION

Interswitch Power Supply

Each Display Unit supplies +12V and -12V to the Interswitch.

The +12V) inputs are diode OR'd to produce   +11V

The -12V) inputs are diode OR'd to produce   -11V

An on-board regulator produces +5V for the logic.

Interswitch Unit Hardware Description

A 89C52 microcontroller with on-board Flash memory running at 12 MHZ communicates with the Display Units via bi-directional serial links. The data transferred includes status i.e. Display is master or slave, which Transceiver is connected to which Display. A watchdog facility is also provided.

A Quad UART is used for the bi-directional serial communication with the Display Units or Compatibility Unit. Received data generates interrupts for the microcontroller which reads the data from the UART and acts upon that data. A message is then returned with the new status. Any change of status is stored in an EEPROM allowing the Interswitch to return to the current status in the event of a power failure.

Logic multiplexers are used to route the signals between the Display Units and Transceivers and analogue 'T' switches route the video.

The microcontroller also drives an LED status display.

ENVIRONMENTAL SPECIFICATION

Temperature

Operating	-15°C to +55°C

Storage	-25°C to +70°C

Relative Humidity

95% at 40°C ( non-condensing )

Weights and Dimensions

|  | Height (mm) | Depth (mm) | Width (mm) | Nominal Weight (kg) |
| --- | --- | --- | --- | --- |
| 2-Way Interswitch Unit 65842F | 346 | 109 | 374 | 5.0 |

Compass Safe Distances

|  | Standard | Steering |
| --- | --- | --- |
| 2-Way Interswitch Unit 65842F | 0.9 m | 0.6 m |

SPARES LIST

| Description | Part Number |
| --- | --- |
| Logic PCB | 65842801 |

# SECTION 3: SCANNER CONTROL UNIT 65427AH

Contents

INTRODUCTION	27

COMMISSIONING	27

SPARES	28

Illustrations

FIGURE 3.10 Scanner Control Unit 65427AH	27

FIGURE 3.11 Internal Connections Scanner Control Unit 65427AH	28

INTRODUCTION

Scanner Control unit 65427AH is identical to Scanner Control unit 65837AH apart from the operating voltage of the contactor. This is 24V dc instead of 12V dc.

For installation and servicing details use Ancillaries Units Manual 6500012 Chapter 6.

COMMISSIONING

Linking.

Link 	Contactor 1 (L1) to Thermal Overload terminal 4 see Figure 3.15

Current Trip Setting.

Set current trip to minimum. i.e. fully anti-clockwise.

FIGURE 3.10 Scanner Control Unit 65427AH

FIGURE 3.11 Internal Connections Scanner Control Unit 65427AH

SPARES

This is a replaceable unit so there is no separate spares listing