CHAPTER 5

ROUTINE MAINTENANCE

CONTENTS

INTRODUCTION	2

WARNINGS AND CAUTIONS	2

MAINTENANCE PLAN	3

# INTRODUCTION

The BridgeMaster E radar system is designed to function reliably for many years. To ensure optimum performance, specific maintenance tasks must be performed at regular intervals.

This chapter details the recommended maintenance plan that should be adhered to.

# WARNINGS AND CAUTIONS

Warning Lethal Voltage Hazard

When access covers are removed, lethal voltages may be exposed. Some capacitors used in the equipment take several minutes to discharge their stored voltages after switch OFF; this is a lethal voltage hazard. Always set the supply switch-fuse to OFF before removing the access covers of the equipment.

Warning Health Hazard

When cleaning the inside of the equipment, take care not to inhale dust. The dust is a temporary health hazard, depending on individual allergies.

Warning Radiation Hazard

Keep outside the hazard zone around an antenna or open waveguide radiating power. When it is necessary to work on the Scanner Unit, make sure that radar is switched OFF, and that the Mains Isolator in the Scanner Control Unit is turned to the OFF position.

Never look directly into an open waveguide.

Radar and other forms of RF radiation can cause Cardiac Pacemakers to malfunction. If you use a Cardiac Pacemaker and suspect a malfunction, leave the vicinity of the radar system immediately and seek medical advice.

Most countries accept that there is no significant radiation hazard at RF power density levels of up to 10 mW/cm2.

Caution Electrostatic Sensitive Devices  (ESSDs)

This equipment contains ESSDs. Take care not to damage these devices by discharge of electrostatic voltages.

# MAINTENANCE PLAN

| Equipment |  |  | Maintenance Task |  | Manpower | ### Tools |  | Spare Parts |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Designation | Part Number | Description | Interval (elapsed time) | Handbook Reference | Duration (note 1) | Type (note 2) | Designation | Part Number | Quantity |  |
| Transceiver Compatibility Unit | 65841F | Check security of fixings, visually inspect external condition. | Yearly |  | 0.1 hours |  |  |  | 1, if required |  |
| Transceiver | 65825CA | Inspect, check fans & clean or replace fan filters. Check security of fixings, and visually inspect external condition. | 3 monthly | 65825010 Chapter 7 | 0.5 hours |  | Filter | 1709992 | 1 or 2, if required |  |
|  |  | Check transmitting hours, and magnetron current. Replace magnetron if necessary. | Yearly |  | 0.5 hours 1 hour if magnetron changed | Digital Multimeter I/P Impedance >10M Ohms | Magnetron | 910/2222 | 1 if required |  |
| #### Display Processor | 65800SAV/1/1 (note 3) | Check fan, clean or replace filter. Charge rechargeable memory cards (if fitted). Check screen for dirt, clean if necessary. Check security of fixings, and visually inspect external condition. | 3 monthly | 65800010B Chapter 7 | 0.5 hours + battery charge 1 hour minimum | Soft cloth anti-static cleaner. Do not use a solvent. | Filter | 65800111 |  | 1 if required |
|  |  | Replace primary battery in memory card (if fitted) | 2 yearly | 65800010B Chapter 7 | 0.1 hour |  | Battery Kit | 65600712 |  | 1 |
| #### Display Monitor | 65821R/MU | Check fan operation. Clean Display screen. Check security of fixings, and visually inspect external condition. | 3 monthly | 65800010B Chapter 7 | 0.25 hours |  |  |  |  |  |
| #### Video Control Unit | 65857A | Check operation of controls. Check security of fixings, and visually inspect external condition. | yearly | 65857010 Chapter 5 | 0.25 hours |  |  |  |  |  |
| #### Isolating Switch | 65800700 | Check security of fixings, visually inspect external condition. | Yearly |  | 0.1 hours |  |  |  |  |  |
| Isolating Transformer | 9640750 | Check security of fixings, visually inspect external condition. | Yearly |  | 0.1 hours |  |  |  |  |  |
| Interswitch Unit | 65842F | Check security of fixings, visually inspect external condition. | Yearly |  | 0.1 hours |  |  |  |  |  |
| Scanner Control Unit | 65427AH | Check security of fixings, visually inspect external condition. | Yearly |  | 0.1 hours |  |  |  |  |  |

Notes:

- All tasks can be carried out by one person unless otherwise indicated.

- Assumes normal hand tools available.

- Display Memory Cards supplied may be either rechargeable or contain primary batteries.  Card maintenance at monthly or 2-yearly intervals as indicated. Battery replacement (where relevant) to be carried out at time of a 3-monthly maintenance.