WARNINGS AND CAUTIONS

WARNING:  LETHAL VOLTAGE HAZARD

When access covers are removed, lethal voltages may be exposed. Some capacitors used in the equipment take several minutes to discharge their stored voltages after switch OFF; this is a lethal voltage hazard. Always set the supply switch-fuse to OFF before removing the access covers of the equipment.

WARNING:  HEALTH HAZARD

When cleaning the inside of the equipment, take care not to inhale dust. The dust is a temporary health hazard, depending on individual allergies.

WARNING:  RADIATION HAZARD

Keep outside the hazard zone around an antenna or an open waveguide radiating power. When it is necessary to work on the Scanner Unit, make sure that the radar is switched OFF, and that the Man Aloft Switch  is turned to the OFF position.

Never look directly into an open waveguide.

Radar and other forms of RF radiation can cause Cardiac Pacemakers to malfunction. If you use a Cardiac Pacemaker and suspect a malfunction, leave the vicinity of the radar system immediately and seek medical advice.

Most countries accept that there is no significant radiation hazard at RF power density levels of up to 10 mW/cm2.

CAUTION:  ELECTROSTATIC SENSITIVE DEVICES (ESSDs)

This equipment contains ESSDs. Take care not to damage these devices by the  discharge of electrostatic voltages.

| Suff | Rev | Issue Date | Date Incorporated | Incorporated By |
| --- | --- | --- | --- | --- |
| 1 | A | March 2005 |  |  |

Technical Addendum NRA2524

OVERALL CONTENTS

Related Documents 	Preamble

Software Licence Agreement	Preamble

System Overview and Technical Specification 	Chapter 1

Installation and Commissioning	Chapter 2

Addenda for Non-Standard units	Chapter 3

Fault Reporting and First Line Servicing	Chapter 4

Routine Maintenance	Chapter 5

Spares Listing 	Chapter 6

#### RELATED DOCUMENTS

- BridgeMaster E Ships Manual	Publication Ref 65800010B

- BridgeMaster E Technical Manual	Publication Ref 65800011

- Ancillary Units & Radar Systems Manual	Publication Ref 65800012

- Special Features Addendum	Publication Ref 65800013

- 25kW I-Band Transceiver Technical Manual	Publication Ref 65825010

- Video Control Unit Technical Manual	Publication Ref 65857010

- User Guide for Combattante III	Publication Ref USG2524

PREFACE

THIS MANUAL

This manual is intended for use by the Installation/Service Engineer. It is intended to be used as a supplement to the manuals listed under Related Documents.

The manual is divided into chapters as follows:

Chapter 1 – General Introduction.  This chapter includes an overview of the system, a schedule of equipment and the interface specifications.

Chapter 2 - Installation and Commissioning.  This chapter includes wiring schedules and commissioning information not included in other manuals.

Chapter 3 – Ancillary Units.  This chapter gives additional information on the Interswitch Unit, Transceiver Compatibility Unit and Scanner Control Unit.

Chapter 4 - Fault Reporting and First Line Servicing.  This chapter gives procedures for fault reporting and fault finding at system level

Chapter 5 - Routine Maintenance.  This chapter gives procedures for routine maintenance.

Chapter 6 – Spares Lists.

NOTICE

This document may not be reproduced either in part or in whole without written permission of Northrop Grumman Sperry Marine.

GLOSSARY OF TERMS

Scanner Unit 		Comprises the Antenna and Turning Unit.

Antenna 		Slotted waveguide array, or horn and parabolic reflector, for transmitting and receiving microwave signals.

Turning Unit 		Contains the Antenna rotation motor, the microwave rotary joint, and an azimuth and heading marker generator.

Bulkhead Transceiver 		Transmitter/Receiver mounted below decks with microwave connection to the Turning Unit.

Display Unit 		The monitor and processor unit including controls.

Scanner Control Unit 		A unit which switches power to the SCOUT system under the control of the Transceiver Compatibility Unit.

Interswitch Unit 		Enables two or  more Transceiver systems to be connected together so that any Display may be connected to any Transceiver Unit.

Display Compatibility Unit 		Converts Transceiver Control messages to a form that can be used by the Video Control Unit.

Listed abbreviations and acronyms.

A			Ampere

AC			Alternating Current

ARPA 			Automatic Radar Plotting Aid

CRT			Cathode Ray Tube

dB			decibel

DC 			Direct Current

ft 			feet

GPS 			Global Positioning System

GMT			Greenwich Mean Time

Hz 			Hertz (unit of Frequency)

I/O 			Input/Output

Kt 			Knot

kW 			Kilowatt

LCD 			Liquid Crystal Display

LED 			Light Emitting Diode

LNFE			Low Noise Front End

nm.			Nautical Mile

NMEA			National Marine Electronic Association

PCB			Printed Circuit Board

PPI			Plan Position Indicator

PSU 			Power Supply Unit

RFI			Radio Frequency Interference

NNF			Not Normally Fitted

rpm 			revolutions per minute

SART			Search And Rescue Transponder

TX/RX			Transmitter/Receiver (Transceiver)

UTC			Coordinated Universal Time

V			Volt

SOFTWARE LICENCE AGREEMENT

When you receive your radar, it will include factory-installed software, the use of which is subject to the following Licence Agreement below.

-----IMPORTANT-----

READ THE LICENCE TERMS PRINTED BELOW BEFORE USING THE EQUIPMENT. USE OF THE EQUIPMENT INDICATES YOUR ACCEPTANCE OF THE TERMS OF THE LICENCE AGREEMENT.

LICENCE AGREEMENT

This legal document is an agreement between you ("the Customer") and Northrop Grumman Sperry Marine B.V. ("the Company").

BY USING THE HARDWARE PRODUCT SYSTEM UPON WHICH THE COMPANY HAS INSTALLED THE SOFTWARE THE CUSTOMER IS AGREEING TO BECOME BOUND BY THE TERMS OF THIS AGREEMENT.

FAILURE TO COMPLY WITH THIS AGREEMENT WILL INVALIDATE THE WARRANTY OF THE PRODUCT SYSTEM UPON WHICH THE COMPANY HAS INSTALLED THE SOFTWARE ("THE SOFTWARE") AND MAY RESULT IN LEGAL PROCEEDINGS BEING TAKEN AGAINST THE CUSTOMER.

THIS LICENCE AGREEMENT SHALL BE GOVERNED CONSTRUED AND SHALL TAKE EFFECT IN ACCORDANCE WITH THE LAWS OF ENGLAND AND SHALL BE SUBJECT TO THE EXCLUSIVE JURISDICTION OF THE ENGLISH COURTS TO WHICH THE CUSTOMER AND THE COMPANY BY AGREEMENT IRREVOCABLY SUBMIT.

THE CUSTOMER IS AUTHORISED TO USE THE SOFTWARE ON THE HARDWARE PRODUCT SYSTEM UPON WHICH IT WAS INSTALLED BY THE COMPANY. THE SOFTWARE MAY NOT BE COPIED OR TRANSFERRED FOR USE ON ANY HARDWARE PRODUCT SYSTEM OTHER THAN THAT ON WHICH IT WAS INSTALLED BY THE COMPANY.

The software is supplied to the Customer. The Company grants the Customer a world-wide royalty-free licence to use the Software provided that:

1.	The Customer shall only operate the Software in conjunction with the hardware product system upon which it was installed by the company. No other use is authorised, and in particular the Customer may not transfer use of the Software to any other BridgeMaster hardware product system owned or used by the Customer.

2.	The Customer shall not reverse assemble modify or decompile the Software except only to the extent expressly permitted by the provisions of the Copyright, Designs and Patents Act 1988.

- The Customer shall not make copies of the Software.

4.	The Customer shall not have any title to or ownership of the Software or copyright or any other intellectual property right in the Software, but shall have ownership of the physical media.

5.	The Customer acknowledges that the Software may in part have been developed by any third party software supplier(s) named in the copyright notice(s) included with the Software and agrees where this is the case that the third party supplier(s) (or the Company on behalf of the third party supplier) shall be entitled to enforce the provisions of this Agreement to the extent reasonably necessary to protect its interest as owner of such third party software.

6.	The Customer shall not rectify or have rectified any defect in the Software. The Customer shall give the Company full details in writing of any serious defect and the Company shall use reasonable endeavours to issue any correction necessary to rectify the defect as soon as reasonably practicable thereafter, at no cost to the Customer.

7.	The Customer shall not sublicense the Software to third parties or supply transfer or assign the Software to third parties.

8.	The Software may be enhanced or corrected from time to time by the Company or by any such third party software supplier referred to above. All such corrections and enhancements shall be subject to the restrictions set out in this Licence Agreement. Any such enhancements shall be supplied at the Company's current rates prevailing at the date of invoice.

9.	The Company may revoke the licence granted to the Customer if the Customer shall go into receivership liquidation, administration, or any other insolvency process, or shall fail to comply with any of these conditions provided that (in the case of any breach capable of remedy) the Customer has been requested by notice in writing to cure the failure and has failed to do so within thirty (30) days of such notice.

10.	The Customer shall ensure the security of the Software and shall not disclose the Software or any parts thereof to any third party not approved beforehand by the Company. Insofar as it is necessary to disclose the Software or aspects thereof to employees or to third parties dealing with Customer in relation to matters for which the Software is used, such disclosure is permitted only to the extent necessary for such purpose and only to the employee or other persons who require to know the same.