2003 ELECTRICAL

Fuses & Circuit Breakers - New Beetle & New Beetle Convertible  
IDENTIFICATION

ENGINE CODE

Engines are identified by a 3-letter code stamped on the engine. For engine code locations, see Fig. 1 -Fig. 3 .  Engine codes are also located on the vehicle data plate.

NOTE:	Engine number for 1.8L turbo can be found on mating flange between engine  
and transmission. Additionally there is a sticker on cylinder head (valve) cover

with engine code and serial number.

Fig. 1: Locating Engine Identification Code (1.8L -AWD Shown; AWV Similar)

Courtesy of VOLKSWAGEN UNITED STATES, INC.

NOTE:	Engine code for 1.9L TDI is located between diesel injection pump and exhaust  
manifold, on engine block. Additionally there is a sticker on the timing belt  
guard containing engine code and serial number. Engine code is also included  
on vehicle data plate.

Fig. 2: Locating Engine Identification Code (1.9L TDI - ALH)  Courtesy of VOLKSWAGEN UNITED STATES, INC.

NOTE:	Engine code and serial number for 2.0L is located can be found on flange  
between engine/transmission. Additionally there is a sticker on the timing belt

guard containing engine code and serial number. Engine code is also included  
on vehicle data plate.

Fig. 3: Locating Engine Identification Code (2.0L - AVH/AZG & BDC)  
Courtesy of VOLKSWAGEN UNITED STATES, INC.

FUSES

FUSE COLOR IDENTIFICATION

FUSE COLOR/AMP RATING

| Fuse Color | Amp Rating |
| --- | --- |
| Beige | 5 |
| Blue | 15 |
| Brown | 7.5 |
| Green | 30 |
| Red | 10 |
| Violet | 3 |

| White | 25 |
| --- | --- |
| Yellow | 20 |

FUSE (HOLDER) PANEL LOCATION

Fuse (holder) panel is located behind trim panel on far left side of dashboard. See Fig. 4 .

Fig. 4: Locating Fuse (Holder) Panel

Courtesy of VOLKSWAGEN UNITED STATES, INC.

Fig. 5: Identifying Fuse Panel Fuse Locations  
Courtesy of AUDI OF AMERICA, INC.

FUSE PANEL IDENTIFICATION

Fuse Bracket/Battery Panel

Fuse bracket/battery panel is located on battery cover. See Fig. 6.

Circuit Protected

Fig. 6: Locating Fuse Bracket/Battery Panel

Courtesy of VOLKSWAGEN UNITED STATES, INC.

Circuit Protected

FUSE IDENTIFICATION (FUSE BRACKET/BATTERY PANEL)

| Fuse No. | Amp Rating |  |
| --- | --- | --- |
| S162 |  |  |
| Engine Codes AVH/AZG | 50 | Secondary Air Injection Pump Relay |
| Engine Code AWV | 50 | Fuse 51 (20 A), Secondary Air Injection Pump Relay |
| Engine Code ALH | 50 | Preheating Coolant (Low & High Heat Output) Relay |
| S163 |  |  |
| Engine Codes AVH/AZG, AWV & BDC |  |  |
|  | 30 | Coolant Fan Control Module |
| AWV | 30 | Third Speed Coolant Fan Control Relay |
| BDC | 40 | Secondary Air Injection Pump Relay |
| S176 | 110 | Load Reduction Relay |
| S177 |  |  |
| All Engine Codes | 110 | Generator |
| BDC | 150 | Generator |
| S178 | 30 | ABS Control Module |

50 	Fuel Pump Relay

Engine Code ALH 	50 	Glow Plug Triggering Control Module  
S164

Engine Code AVH/AZG &  
ALH

| S179 | 30 | ABS Control Module |
| --- | --- | --- |
| S180 | 30 | Coolant Fan Control Thermal Switch |

Fuse (Holder) Panel

For fuse position number, amp rating and description of circuits protected, see FUSE IDENTIFICATION  (FUSE HOLDER PANEL) table.

NOTE:	Fuses 23 through 44 may be designated in wiring diagrams with a prefix of 2.  
For example, fuse No. 29 may be labeled as 229. Fuse No. 43 may be labeled as

243.

FUSE IDENTIFICATION (FUSE HOLDER PANEL)

Fuse No. 	Amp  
Rating

|  |  | Circuit Protected |
| --- | --- | --- |
| 1 | 10 | Glove Compartment Light Switch, Glove Compartment Light, Washer Nozzle Heater (Left & Right), Heated Mirror Relay |
| 2 | 10 | Turn Signal Relay |
| 3 | 5 | Instrument Panel Light Dimmer Switch |
| 4 | 5 | Dual Horns, License Plate Light |
| 5 | 7.5 | Clutch Vacuum Vent Valve Switch, Power Mirror Switch, Driver's Heated Seat Adjust Control Switch, Automatic Day/Night Interior Mirror, Cruise Control Switch, A-C High Pressure Sensor, Fresh Air/Recirculation Air Switch, (1) Brake Vacuum Vent Switch, (2)Brake Pedal Switch |
| 6 | 5 | Comfort System Central Control Module, Driver's Door Warning Indicator Light |
| 7 | 10 | Back-Up Light Switch, Park Neutral Position Relay, Selector Lever Position Warning Light, Speedometer Vehicle Speed Sensor, (3) Multi-Function Transmission Range Switch |
| 8 | N/A | Vacant |
| 9 | 5 | ABS Control Module, Anti-Slip Control Switch, Steering Angle Sensor |
| 10 |  |  |
| All Engine Codes |  |  |
|  | 10 | Diesel Direct Fuel Injection ECM |
| 11 | 5 | Instrument Panel, Instrument Panel Control Indicator Unit, Shift Lock Solenoid |
| 12 | 7.5 | DLC (Terminal 16) |

10 	ECM, ECM Power Relay

Engine Code  ALH

| 13 | 10 | High Mount Brake Light, Left Brake/Tail Light, Brake Light Switch |
| --- | --- | --- |
| 14 | 10 | Comfort System Central Control Module |
| 15 | 5 | Control Module W/Indicator, Transmission Control Module, ABS Warning Indicator Light |
| 16 | 10 | Coolant Fan Control Module |
| 17 | N/A | Vacant |
| 18 | 10 | Headlight High Beam Indicator |
| 19 | 10 | Headlight (High Beam - Left) |
| 20 | 15 | Headlight (High & Low Beam - Right) |
| 21 | 15 | Daytime Running Lights Change-Over Relay, Headlight (Low Beam - Left) |
| 22 | 5 | Control Module W/Indicator, Parking Light (Right), Front Side Marker Light (Right), Tail Light (Right), Brake Light (Right ) Turn Signal Light (Right Front) |
| 23 | 5 | Control Module W/Indicator, Tail Light (Left), Parking Light (Left), Front Side Marker Light (Left), Brake Light (Left), Turn Signal (Left), Headlight Switch |
| 24 | 20 | Wiper/Washer Intermittent Relay & Switch |
| 25 | 25 | Fresh Air Blower Switch, Fresh Air Blower Motor |
| 26 | 20 | Rear Window Defogger Switch |
| 27 | N/A | Vacant |
| 28 | 15 | Fuel Pump |
| 29 |  |  |
| All Engine Codes |  |  |
|  | 10 | Diesel Direct Fuel Injection ECM |
| 30 | 20 | Power Sunroof Control Module |
| 31 | 20 | Transmission Control Module, Multi-Function Transmission Range Switch, Brake Booster Control Module |
| 32 |  |  |
| All Engine Codes |  |  |
|  | 15 | Diesel Direct Fuel Injection ECM, Modulating Piston Displacement Sensor, PCV Heating Element |
| 33 | 20 | Wiper/Washer Intermittent Relay |
| 34 |  |  |
| Engine Codes AVH/AZG & BDC |  |  |

15 	ECM

Engine Code  ALH

10 	Fuel Injectors

Engine Code  ALH

10 	PCV Heating Element

Engine Code  ALH

| Engine Code AWV | 10 | Turbocharger Recirculation Valve, Wastegate Bypass Valve, Camshaft Adjuster Valve |
| --- | --- | --- |
|  | 10 | Wastegate Bypass Regulator Valve, EGR Vacuum Regulator Solenoid Valve, Intake Manifold Flap Change-Over Valve, Cold Start Injector, Glow Plug Triggering Control Module |
| 35 | 30 | (4)Luggage Compartment 12V Outlet |
| 36 | 15 | Fog Light Switch |
| 37 | 10 | Comfort System Central Control Module, Instrument Cluster, Radio |
| 38 | 15 | Remote Fuel Door Switch & Motor, Comfort System Central Control Module |
| 39 | 15 | Turn Signal Relay, Side Door Control Module (Driver's & Passenger's) |
| 40 | 20 | Dual Horn Relay |
| 41 | 15 | Cigarette Lighter, Cigarette Lighter Light, 12V Socket, (5) Luggage Compartment 12V Outlet, Lighting Controller Illumination, Rear Window Defogger Switch Illumination |
| 42 | 25 | Radio, Amplifier, Emergency Flasher |
| 43 |  |  |
| All Engine Codes |  |  |
|  | 10 | Brake Vacuum Vent Valve Solenoid, Brake Pedal Switch |
| 44 | 15 | Heated Seat Control Module (Driver's & Passenger's) |
| 67 | 30 | Convertible Top Control Module |
| 68 | 30 | Convertible Top Hydraulic Pump Relay Circuit Breaker |
| (1)On 1.8L (AWV) engines only. (2)On 2.0L (AVH/AZG and BDC) engines only. (3)On 2.0L Engine (BDC) only. (4)On Convertible only. (5)Except Convertible. |  |  |

10 Brake Vacuum Vent Valve Solenoid, Leak Detection Pump,  Heated Oxygen Sensors Pre & Post Catalytic Converter,  
EVAP Canister Purge Regulator Valve

Engine Code  ALH

MISCELLANEOUS FUSES

| Fuse No. | Amp Rating | Circuit Protected |
| --- | --- | --- |
| S51 (1) | 20 | Brake System Vacuum Pump (Automatic Only) |
| S329 (2) | 30 | Rear Spoiler Motor |

(1)Located in left side of engine compartment, within protective housing.

(2)Only equipped on 1.8L GL.

RELAYS

RELAY PANEL & THIRTEENFOLD AUXILIARY RELAY PANEL

NOTE:	Relay panels are under driver's side of dash, behind trim panel.

For ECM power, secondary air injection pump relays and relay for preheating coolant (high and low output)  
locations, see Fig. 7 , Fig. 8 or Fig. 9 . For relay positions, see Fig. 10 .

Thirteenfold auxiliary relay panel is located above relay panel. For relay number, position number and circuits  protected, see RELAY IDENTIFICATION (RELAY PANEL) and RELAY IDENTIFICATION  
(THIRTEENFOLD AUXILIARY RELAY PANEL) table.

Fig. 7: Locating ECM Power Supply Relay

Courtesy of VOLKSWAGEN UNITED STATES, INC.

Fig. 8: Locating Secondary Air Injection Pump Relay  Courtesy of VOLKSWAGEN UNITED STATES, INC.

Fig. 9: Locating Relay For Preheating Coolant (High & Low Heat Output - 1.9L TDI ALH)  Courtesy of VOLKSWAGEN UNITED STATES, INC.

Fig. 10: Identifying Relay Panel Components

Courtesy of VOLKSWAGEN UNITED STATES, INC.

RELAY IDENTIFICATION (RELAY PANEL)

| Position No. | Relay No. | Circuit Protected |
| --- | --- | --- |
| 1 | J4 | Dual Horn |
| 2 | J59 | Load Reduction |
| 3 | N/A | Not Used |
| 4 | J17 | Fuel Pump |
| V | J31 | Wiper/Washer Intermittent |
| VI | J31 | Wiper/Washer Intermittent |

RELAY IDENTIFICATION (THIRTEENFOLD AUXILIARY RELAY PANEL)

| Position No. | Relay No. | Circuit Protected |
| --- | --- | --- |
| 1 | N/A | Not Used |
| 2 | N/A | Not Used |
| 3 | N/A | Not Used |

| 4 | J434 | Lock Relay For Starter (Clutch Pedal Switch) |
| --- | --- | --- |
| 5 | N/A | Not Used |
| 6 | N/A | Not Used |
| 7 | J89 | Daytime Running Light Change-Over |
| 8 | J99 | (1)Heated Exterior Mirror |
| 9 | N/A | Not Used |
| 10 |  |  |
| J52 |  | (2)Glow Plug |
| J135 |  | (3)Third Speed Coolant Fan Control |
| 11 | J226 | Park/Neutral Position |
| 12 | J317 | (2)Power Supply (Terminal 30, B+) |
| 13 | N/A | Not Used |
| (1)On models equipped with manual windows. (2)Applies to vehicles equipped with 1.9L (ALH) only. (3)On 1.8L (AWV) only. |  |  |

NOTE:	Fuse panel is located above thirteenfold auxiliary relay panel. For positions on  
thirteenfold auxiliary relay panel, see Fig. 10 .

FUSE IDENTIFICATION (THIRTEENFOLD AUXILIARY RELAY PANEL)

| Position No. (1) | Fuse No. | Amp Rating | Circuit Protected |
| --- | --- | --- | --- |
| B (2) | S283 | 15 | Brake Vacuum Pump |
| E | S43 | 30 | Power Window Circuit Breaker |
| F | S111 | 15 | Alarm System & Anti-Theft |
| G | S144 | 15 | Central Locking/Anti-Theft Warning System |
| (1)Manufacturer does not specify position of fuse. Available position is "A", "C" and "D". (2)On convertible models only. |  |  |  |